Configure IPv6 AddressingPending

IPv6 Networking Assignment – Complete Answers

This assignment covers static IPv6 configuration, Unique Local Addresses (ULA), temporary IPv6 addresses, a restaurant-search script, and IPv6 subnetting.

Since you use Windows 11 and Cisco Packet Tracer for networking practice, the steps below focus on Windows CMD and PowerShell, with Linux commands where useful. You can use the answers as your practical record and save the documentation in a text file.

1. Assign a static IPv6 address

Aim: Configure a static IPv6 address on the primary network adapter and verify it.

Windows 11 steps

- Open CMD as Administrator.

- Find your network adapter name:

netsh interface ipv6 show interfaces

3. Check the adapter's current configuration:

ipconfig /all

4. Choose your active adapter name, such as Ethernet or Wi-Fi. For a lab, you can use the documentation IPv6 prefix 2001:db8:1::/64, which is reserved for examples and is not a public internet address.

Run the following in PowerShell as Administrator, replacing Ethernet with your actual adapter name:

New-NetIPAddress `

-InterfaceAlias "Ethernet" `

-IPAddress "2001:db8:1::10" `

-PrefixLength 64

5. Verify the configuration:

ipconfig

Or in PowerShell:

Get-NetIPAddress -AddressFamily IPv6

Expected lab configuration:

| Setting | Value |
| --- | --- |
| Adapter | Ethernet (example) |
| IPv6 address | 2001:db8:1::10 |
| Prefix length | /64 |
| Subnet mask equivalent | ffff:ffff:ffff:ffff:: |

Important: Do not apply the example address to your everyday internet-connected adapter unless your network is configured for it. For a safe lab, use an isolated virtual machine or a test network. A static IPv6 address alone does not guarantee internet connectivity; the network also needs suitable routing and, if required, a default gateway.

2. Configure a Unique Local IPv6 Address (ULA) and test connectivity

Aim: Assign an IPv6 address starting with fd and ping another device on the same LAN.

ULA addresses are private IPv6 addresses intended for local communication. They are not globally routable on the public internet.

Example network

| Device | IPv6 address | Prefix |
| --- | --- | --- |
| Laptop 1 | fd12:3456:789a:1::10 | /64 |
| Laptop 2 | fd12:3456:789a:1::20 | /64 |

Windows steps

Open PowerShell as Administrator on Laptop 1:

New-NetIPAddress -InterfaceAlias "Ethernet" -IPAddress "fd12:3456:789a:1::10" -PrefixLength 64

On Laptop 2, configure the second address:

New-NetIPAddress -InterfaceAlias "Ethernet" -IPAddress "fd12:3456:789a:1::20" -PrefixLength 64

Replace Ethernet with the adapter actually in use. Both devices must be connected to the same LAN, and the addresses must not conflict with existing addresses.

Verify the configuration:

ipconfig

Test connectivity from Laptop 1 to Laptop 2:

ping -6 fd12:3456:789a:1::20

Expected result: Replies from the second device indicate successful IPv6 communication. If ping fails, check the adapter, prefix, firewall rules, and whether both devices share the same Layer 2 network.

Linux alternative

Replace eth0 with your actual interface name, found using ip link.

sudo ip -6 addr add fd12:3456:789a:1::10/64 dev eth0

ip -6 addr show dev eth0

ping -6 fd12:3456:789a:1::20

These Linux ip commands generally configure a runtime address that does not persist after reboot.

3. Configure and observe a temporary IPv6 address

Aim: Observe how an automatically generated temporary IPv6 address behaves after a network reconnect or reboot.

A temporary IPv6 address is commonly generated by IPv6 privacy extensions. It can help reduce the use of a stable interface identifier when making outgoing connections. Its exact behavior depends on the operating system and network configuration.

Windows 11 steps

- Open PowerShell as Administrator.

- Enable IPv6 privacy addressing:

netsh interface ipv6 set privacy state=enabled

- Disconnect and reconnect Wi-Fi, or restart the PC.

- Run:

ipconfig /all

Or:

Get-NetIPAddress -AddressFamily IPv6 |

Format-Table InterfaceAlias,IPAddress,AddressState,PrefixOrigin,SuffixOrigin,PreferredLifetime

5. Record the IPv6 addresses and their lifetimes before and after reconnecting.

Observation table for your practical record

| Test | What to record |
| --- | --- |
| Before reconnecting | Temporary IPv6 address shown by the system |
| After reconnecting | Whether the address remains or changes |
| After reboot | Whether a new temporary address appears |
| Conclusion | Address behavior observed on your machine |

Important: Do not write that the address definitely changed unless you actually observed it. A temporary address may remain valid after a reconnect, or a new one may be generated. Record the real output from your system.

4. Create a Zomato-style restaurant search script

Aim: Store restaurant names and their IPv6 addresses in an array, accept a partial restaurant name from the user, and return matching IPv6 addresses.

We will use Python because it is simple to run on Windows or Linux.

Python program

Save the following code as restaurant_ipv6.py.

restaurants = [    {"name": "Pizza Palace", "ipv6": "fd12:3456:789a:1::101"},    {"name": "Burger Hub", "ipv6": "fd12:3456:789a:1::102"},    {"name": "Spice Garden", "ipv6": "fd12:3456:789a:1::103"},    {"name": "Royal Dosa", "ipv6": "fd12:3456:789a:1::104"},    {"name": "Food Junction", "ipv6": "fd12:3456:789a:1::105"}]search_name = input("Enter restaurant name: ").strip().lower()matches = [    restaurant for restaurant in restaurants    if search_name in restaurant["name"].lower()]if matches:    print("\nMatching restaurants:")    for restaurant in matches:        print("Restaurant:", restaurant["name"])        print("IPv6 Address:", restaurant["ipv6"])else:    print("No matching restaurant found.")

How to execute the program

- Install Python if it is not already installed.

- Save the code in a file named restaurant_ipv6.py.

- Open CMD in the folder where you saved the file.

- Run:

python restaurant_ipv6.py

Sample output

Example terminal session

Enter restaurant name: pizza

Matching restaurants:
Restaurant: Pizza Palace
IPv6 Address: fd12:3456:789a:1::101

Explanation:

- The list stores restaurant names and example IPv6 addresses.

- input() accepts the user's search.

- lower() makes the search case-insensitive.

- The in operator finds partial name matches.

- The program prints each matching restaurant and its IPv6 address.

These are simulated addresses for the assignment, not verified addresses of real restaurant servers.

5. IPv6 subnetting scenario and manual address assignment

Aim: Create at least three IPv6 subnets using an AI-generated scenario, assign one valid address to a computer, and document the configuration.

AI-generated scenario

Suppose a food-delivery company has one private IPv6 network for its office, restaurant services, and customer support systems.

The company has the Unique Local Address prefix:

fd12:3456:789a::/48

Divide it into three /64 subnets:

| Department | Subnet | Example computer address |
| --- | --- | --- |
| Office network | fd12:3456:789a:1::/64 | fd12:3456:789a:1::10 |
| Restaurant services | fd12:3456:789a:2::/64 | fd12:3456:789a:2::10 |
| Customer support | fd12:3456:789a:3::/64 | fd12:3456:789a:3::10 |

Each subnet uses a /64 prefix, which is the standard size for most IPv6 LANs. The three subnets are distinct because their fourth hexadecimal groups differ.

Manually assign an address on Windows

For this example, use the office network on an isolated lab adapter or a suitable test network. If you already assigned the same address in Task 2, use that configuration rather than adding it again.

Open PowerShell as Administrator:

New-NetIPAddress `

-InterfaceAlias "Ethernet" `

-IPAddress "fd12:3456:789a:1::10" `

-PrefixLength 64

Verify the address:

Get-NetIPAddress -InterfaceAlias "Ethernet" -AddressFamily IPv6

Or:

ipconfig

Expected result: The adapter should show fd12:3456:789a:1::10 with prefix length 64, provided the configuration was accepted and duplicate-address detection succeeded.

Linux alternative

sudo ip -6 addr add fd12:3456:789a:1::10/64 dev eth0

ip -6 addr show dev eth0

Replace eth0 with the correct interface name.

Download your practical documentation

I have prepared a text file containing all five tasks, commands, the Python script, the subnetting scenario, and an observation template for Task 3.

IPv6 Networking Assignment.txt

All 5 practical tasks · Editable text file

Download the IPv6 assignment text file

Before submitting: Run the commands on your own system, record the actual output for Tasks 1–3 and 5, and add screenshots if your teacher requires practical evidence. The expected results in this answer are examples, not claims that the commands have already been executed on your computer.

Reference: Microsoft Learn — New-NetIPAddress.

Microsoft Learn

IPv6_Networking_Assignment.txtDocument