Ethernet standard and specification Pending

Ethernet Cabling Assignment – Detailed Answers

1. RJ-45 Connector Pin Layout – T568A and T568B

An RJ-45 Ethernet connector has 8 pins. The two common wiring standards are T568A and T568B.

RJ-45 Pin Diagram

When looking at the front of the RJ-45 plug, with the gold contacts facing you and pin 1 on the left:

RJ-45 CONNECTOR

___________________

| 1 2 3 4 5 6 7 8 |

|___________________|

↓ ↓ ↓ ↓ ↓ ↓ ↓ ↓

Pin numbers

T568A:                         T568B:

1  White/Green                1  White/Orange

2  Green                      2  Orange

3  White/Orange               3  White/Green

4  Blue                       4  Blue

5  White/Blue                 5  White/Blue

6  Orange                     6  Green

7  White/Brown                7  White/Brown

8  Brown                      8  Brown

Color Code Table

| Pin | T568A | T568B |
| --- | --- | --- |
| 1 | White/Green | White/Orange |
| 2 | Green | Orange |
| 3 | White/Orange | White/Green |
| 4 | Blue | Blue |
| 5 | White/Blue | White/Blue |
| 6 | Orange | Green |
| 7 | White/Brown | White/Brown |
| 8 | Brown | Brown |

Easy way to remember T568B

1 → White/Orange

2 → Orange

3 → White/Green

4 → Blue

5 → White/Blue

6 → Green

7 → White/Brown

8 → Brown

Important: For a normal straight-through Ethernet cable, the same standard must be used on both ends, for example:

T568B                         T568B

End A                         End B

1 White/Orange  ────────────  1 White/Orange

2 Orange        ────────────  2 Orange

3 White/Green   ────────────  3 White/Green

4 Blue          ────────────  4 Blue

5 White/Blue    ────────────  5 White/Blue

6 Green         ────────────  6 Green

7 White/Brown   ────────────  7 White/Brown

8 Brown         ────────────  8 Brown

If one end is T568A and the other is T568B, the transmit and receive pairs are crossed, producing a crossover cable.

2. Comparison of 10BASE-T, 100BASE-TX and 1000BASE-T

| Feature | 10BASE-T | 100BASE-TX | 1000BASE-T |
| --- | --- | --- | --- |
| Ethernet speed | 10 Mbps | 100 Mbps | 1000 Mbps / 1 Gbps |
| Common name | Ethernet | Fast Ethernet | Gigabit Ethernet |
| Cable type | Twisted-pair copper | Twisted-pair copper | Twisted-pair copper |
| Typical cable category | Cat3 or better | Cat5 or better | Cat5e or better |
| Maximum cable length | 100 m | 100 m | 100 m |
| Connector | RJ-45 / 8P8C | RJ-45 / 8P8C | RJ-45 / 8P8C |
| Number of pairs used | 2 pairs | 2 pairs | 4 pairs |
| Typical use | Older/legacy networks | Older office/home networks | Modern LANs, PCs, switches |
| Relative speed | Slow | Faster | Much faster |

Explanation

A. 10BASE-T

10BASE-T is an older Ethernet standard providing a speed of approximately 10 Mbps.

- Uses twisted-pair copper cable.

- Commonly associated with Cat3 or better cable.

- Maximum Ethernet segment length is 100 metres.

- Uses an 8P8C modular connector, commonly called RJ-45.

- It is mostly found in older or legacy networking equipment.

B. 100BASE-TX

100BASE-TX is also called Fast Ethernet.

- Provides a speed of 100 Mbps.

- Uses twisted-pair copper cable.

- Normally uses Cat5 or better cable.

- Maximum cable length is 100 metres.

- Uses an RJ-45/8P8C connector.

- It was widely used in older computers, switches and network devices.

C. 1000BASE-T

1000BASE-T is commonly called Gigabit Ethernet.

- Provides 1 Gbps (1000 Mbps) speed.

- Uses twisted-pair copper cable.

- Cat5e or better is commonly used.

- Maximum cable length is 100 metres.

- Uses RJ-45/8P8C.

- Uses all four twisted pairs for transmission.

- It is very common in modern computers, routers and network switches.

Speed comparison

10BASE-T       → 10 Mbps

100BASE-TX     → 100 Mbps

1000BASE-T     → 1000 Mbps / 1 Gbps

So:

1000BASE-T is 100 times faster than 10BASE-T and 10 times faster than 100BASE-TX.

3. Connecting Two Laptops Directly Using Ethernet

Answer: Use a crossover cable for traditional Ethernet equipment.

If two laptops need to be connected directly to each other without a switch or router, traditionally a crossover Ethernet cable would be used.

Why?

Ethernet devices have separate transmit (TX) and receive (RX) pairs.

For two similar devices, such as:

Laptop A                     Laptop B

TX  ───────────────────────→ RX

RX  ←─────────────────────── TX

The transmitting pair of one laptop needs to connect to the receiving pair of the other laptop.

A crossover cable achieves this by using:

T568A                         T568B

TX  ───────────────────→  RX

RX  ←───────────────────  TX

In a typical 10/100 Mbps Ethernet connection:

Pins 1,2 = one pair

Pins 3,6 = another pair

The crossover wiring swaps these transmit/receive pairs.

Straight-through vs Crossover

| Cable | Wiring | Traditional use |
| --- | --- | --- |
| Straight-through | Same standard on both ends, e.g. T568B–T568B | PC → Switch, Router → Switch |
| Crossover | T568A on one end and T568B on the other | PC → PC, Switch → Switch, traditionally |

Important modern point

Most modern laptops, network cards and switches support Auto-MDI/MDI-X. This allows the Ethernet interface to automatically detect and correct the transmit/receive arrangement.

Therefore, many modern laptops can connect directly using an ordinary straight-through Ethernet cable.

For an assignment, the traditional answer is:

A crossover cable is used to directly connect two similar Ethernet devices such as two laptops because it crosses the transmit and receive pairs. However, modern Ethernet adapters commonly support Auto-MDI/MDI-X, so a straight-through cable may also work.

4. Three Real-World Devices Using Ethernet

Here are three examples that can commonly be found in a home or college network.

| Device | Ethernet connection | Likely Ethernet standard | Typical cable |
| --- | --- | --- | --- |
| Desktop/Lab PC | Ethernet port → switch/router | 1000BASE-T | Cat5e/Cat6 |
| Network Switch | Switch → PCs/router/access points | 1000BASE-T | Cat5e/Cat6 |
| Wi-Fi Router | Router LAN port → PC/switch | 1000BASE-T or 100BASE-TX | Cat5e/Cat6 |

1. Desktop Computer

A desktop computer in a college computer lab may have an Ethernet/LAN port.

Desktop PC

│

│ Cat5e/Cat6 Ethernet

↓

Network Switch

Likely standard: 1000BASE-T

Modern desktop computers commonly support Gigabit Ethernet, giving speeds up to 1 Gbps on suitable equipment and cabling.

2. Network Switch

A network switch is commonly used in college computer labs to connect many computers.

PC 1 ───┐

PC 2 ───┤

PC 3 ───┤

PC 4 ───┤

↓

Ethernet Switch

│

↓

Router

Likely standard: 1000BASE-T

A Gigabit switch can provide up to 1 Gbps per Ethernet port, depending on the particular model.

3. Wi-Fi Router

A home or college router usually has several LAN Ethernet ports.

Internet

│

↓

Router

┌─────┼─────┐

↓     ↓     ↓

PC   Switch  TV

Likely standard: 1000BASE-T on many modern routers.

The router's Ethernet LAN port can connect to a desktop PC, switch, smart TV, or another network device using Cat5e/Cat6 cable.

Final Summary

RJ-45 has 8 pins

↓

T568A / T568B color standards

↓

Straight-through = same standard on both ends

Crossover       = T568A on one end + T568B on other

↓

10BASE-T   = 10 Mbps

100BASE-TX = 100 Mbps

1000BASE-T = 1 Gbps

↓

Maximum copper Ethernet length = 100 m

↓

Common connector = RJ-45 / 8P8C

Key point for exams:
T568B is the most commonly encountered wiring order for Ethernet patch cables. 1000BASE-T provides 1 Gbps over four twisted pairs and can operate up to 100 m over suitable copper cabling.