Day 4: Hands-on Subnetting Practical

IPv4 Subnetting Assignment — 192.168.10.0/24

1. Divide the network into 4 equal subnets

Given network: 192.168.10.0/24

We need to create 4 equal subnets. Each subnet will have the same number of IP addresses.

Calculation

- Original subnet mask: 255.255.255.0 (/24)

- Required subnets: 4

- Borrowed bits: 2, because \(2^2=4\)

- New prefix: /24 + 2 = /26

- New subnet mask: 255.255.255.192

- Total addresses per subnet: \(2^{32-26}=64\)

- Usable host addresses per subnet: \(64-2=62\)

The subnet increment is \(256-192=64\), so the network addresses increase by 64 in the last octet.

Subnet table

| Subnet | Network address | Usable host range | Broadcast address |
| --- | --- | --- | --- |
| 1 | 192.168.10.0/26 | 192.168.10.1 – 192.168.10.62 | 192.168.10.63 |
| 2 | 192.168.10.64/26 | 192.168.10.65 – 192.168.10.126 | 192.168.10.127 |
| 3 | 192.168.10.128/26 | 192.168.10.129 – 192.168.10.190 | 192.168.10.191 |
| 4 | 192.168.10.192/26 | 192.168.10.193 – 192.168.10.254 | 192.168.10.255 |

Subnet mask for all four subnets: 255.255.255.192

2. Assign valid IP addresses to 5 devices

We will use Subnet 1: 192.168.10.0/26.

Its usable range is 192.168.10.1 to 192.168.10.62. We will assign the first five usable IP addresses to the devices.

| Device | Assigned IP address | Subnet mask |
| --- | --- | --- |
| Router (LAN interface) | 192.168.10.1 | 255.255.255.192 |
| Laptop 1 | 192.168.10.2 | 255.255.255.192 |
| Laptop 2 | 192.168.10.3 | 255.255.255.192 |
| Printer | 192.168.10.4 | 255.255.255.192 |
| Mobile | 192.168.10.5 | 255.255.255.192 |

- Default gateway: 192.168.10.1 for the laptops, printer, and mobile.

- Network address: 192.168.10.0 — do not assign to a device.

- Broadcast address: 192.168.10.63 — do not assign to a device.

- Usable hosts remaining: 57 after these five device addresses and the router address are assigned? More precisely, there are 62 usable addresses total, of which 5 are assigned, leaving 57 unused usable addresses.

3. Subnet plan diagram

The diagram below shows all four subnets and the devices connected to Subnet 1.

Main network

192.168.10.0/24

Subnet mask: 255.255.255.0

Subnet 1

192.168.10.0/26

Hosts: .1 – .62

Broadcast: .63

Router: .1

Laptop 1: .2

Laptop 2: .3

Printer: .4

Mobile: .5

Subnet 2

192.168.10.64/26

Hosts: .65 – .126

Broadcast: .127

Other devices can connect here.

Subnet 3

192.168.10.128/26

Hosts: .129 – .190

Broadcast: .191

Other devices can connect here.

Subnet 4

192.168.10.192/26

Hosts: .193 – .254

Broadcast: .255

Other devices can connect here.

For a physical or Packet Tracer diagram, connect the router's LAN interface to a switch, then connect Laptop 1, Laptop 2, and the printer to the switch. Connect the mobile through a wireless access point connected to that LAN. Subnets 2–4 would require their own appropriately configured interfaces or VLANs and routing if they need to communicate with Subnet 1.

4. Step-by-step subnetting calculation for beginners

Step 1: Understand the original network

The given IP address is 192.168.10.0/24.

The /24 means that 24 bits are used for the network portion and 8 bits remain for hosts.

Original subnet mask: 255.255.255.0

Step 2: Calculate the number of borrowed bits

We need 4 subnets.

Use the formula:

\[ 2^n \geq \text{Required subnets} \]

\[ 2^2=4 \]

Therefore, borrow 2 bits from the host portion.

Step 3: Calculate the new prefix and subnet mask

\[ /24+2=/26 \]

The new subnet mask is:

255.255.255.192

The last octet is 192 because the first two borrowed bits have values 128 and 64:

\[ 128+64=192 \]

Step 4: Calculate the number of addresses and usable hosts

There are 6 host bits left:

\[ 32-26=6 \]

Total addresses per subnet:

\[ 2^6=64 \]

Usable host addresses:

\[ 64-2=\boxed{62} \]

We subtract two addresses because one is the network address and the other is the broadcast address.

Step 5: Find the subnet increment

Use:

\[ 256-192=64 \]

Therefore, the subnet network addresses increase by 64:

- First subnet: 192.168.10.0

- Second subnet: 192.168.10.64

- Third subnet: 192.168.10.128

- Fourth subnet: 192.168.10.192

Step 6: Find the broadcast addresses

The broadcast address is one less than the next subnet's network address.

For example:

- Subnet 1: next network .64, so broadcast .63

- Subnet 2: next network .128, so broadcast .127

- Subnet 3: next network .192, so broadcast .191

- Subnet 4: final broadcast .255

Step 7: Assign IP addresses to devices

Choose IP addresses from the usable range of Subnet 1.

- Router: 192.168.10.1

- Laptop 1: 192.168.10.2

- Laptop 2: 192.168.10.3

- Printer: 192.168.10.4

- Mobile: 192.168.10.5

Use subnet mask 255.255.255.192 for all five devices.

Final conclusion

The network 192.168.10.0/24 has been divided into 4 equal /26 subnets. Each subnet contains 64 total IP addresses and 62 usable host addresses. Five devices have been assigned valid IP addresses from the first subnet without using its network or broadcast address.

Is this conversation helpful so far?