IP addressing & Subnetting Pending

IP Address Classes and Subnetting – Detailed Answers

1. IP Address Classes A, B, C, D and E

IPv4 addresses are traditionally divided into five classes: A, B, C, D and E. Classful addressing is mostly historical today because modern networks use CIDR (Classless Inter-Domain Routing), but these classes are still important for understanding basic networking.

| Class | First Octet Range | Default Subnet Mask | Main Purpose | Real-World Example |
| --- | --- | --- | --- | --- |
| Class A | 1–126 | 255.0.0.0 (/8) | Very large networks | Large organization's private network such as 10.0.0.0/8 |
| Class B | 128–191 | 255.255.0.0 (/16) | Medium/large networks | College network such as 172.16.0.0/16 |
| Class C | 192–223 | 255.255.255.0 (/24) | Small networks | Home/office network such as 192.168.1.0/24 |
| Class D | 224–239 | Not applicable | Multicast | Multicast services/devices using addresses such as 224.0.0.1 |
| Class E | 240–255 | Not applicable | Experimental/reserved | Experimental/research use, not normally assigned to ordinary devices |

Class A

Class A addresses have a first octet from 1 to 126.

Example:

10.0.0.0

The private range 10.0.0.0/8 is commonly used inside large private networks.

Class B

Class B addresses have a first octet from 128 to 191.

Example:

172.16.0.0

The range 172.16.0.0–172.31.255.255 is private IPv4 space.

Class C

Class C addresses have a first octet from 192 to 223.

Example:

192.168.1.0

This is commonly used for home networks.

Class D

Class D is used for multicast rather than normal host addressing.

Example:

224.0.0.1

It represents a multicast group used for local-network multicast communication.

Class E

Class E addresses are from 240.0.0.0 to 255.255.255.255 and are reserved for experimental/special purposes rather than normal host assignment.

2. IP Address: 192.168.10.45 / 255.255.255.0

Given

IP address:

192.168.10.45

Subnet mask:

255.255.255.0

The subnet mask is equivalent to:

/24

So we have:

192.168.10.45/24

Step 1 – Convert the mask into binary

Subnet mask:

255.255.255.0

Binary:

255       255       255       0

11111111.11111111.11111111.00000000

Therefore:

Network bits = 24

Host bits    = 8

Step 2 – Find the Network Address

The IP address is:

192.168.10.45

With a /24 mask, the first three octets are the network portion and the last octet is the host portion.

Therefore:

Network portion = 192.168.10

Host portion    = 45

Set all host bits to 0:

192.168.10.00000000

Therefore:

Network Address

192.168.10.0

Step 3 – Find the Broadcast Address

For the broadcast address, set all host bits to 1.

There are 8 host bits:

11111111 = 255

Therefore:

192.168.10.255

Broadcast Address

192.168.10.255

Step 4 – Find Valid Host Range

The network address cannot normally be assigned to a host.

The broadcast address also cannot normally be assigned to a host.

Therefore:

Network address = 192.168.10.0

First host      = 192.168.10.1

Last host       = 192.168.10.254

Broadcast       = 192.168.10.255

Final Answer

| Item | Address |
| --- | --- |
| IP Address | 192.168.10.45 |
| Subnet Mask | 255.255.255.0 |
| CIDR | /24 |
| Network Address | 192.168.10.0 |
| First Valid Host | 192.168.10.1 |
| Last Valid Host | 192.168.10.254 |
| Broadcast Address | 192.168.10.255 |
| Number of usable hosts | 254 |

Host calculation

There are 8 host bits:

2⁸ = 256 total addresses

Two addresses are reserved:

256 - 2 = 254 usable host addresses

So the valid host range is:

192.168.10.1 – 192.168.10.254

3. Subnetting Plan for a Food Delivery Startup

The startup needs 4 separate subnets:

- Delivery Partners

- Restaurants

- Customers

- Admins

Given network:

10.0.0.0/24

We need at least 4 subnets.

Step 1 – Calculate required subnet bits

Formula:

Number of subnets = 2ⁿ

We need 4:

2² = 4

Therefore, we borrow 2 host bits.

Original:

/24

New prefix:

24 + 2 = /26

New subnet mask

/26 = 255.255.255.192

Step 2 – Calculate hosts per subnet

A /26 leaves:

32 - 26 = 6 host bits

Total addresses:

2⁶ = 64

Usable hosts:

64 - 2 = 62

Therefore, each subnet has 62 usable host addresses.

Step 3 – Find the subnets

With /26, the block size is:

256 - 192 = 64

Therefore, the subnets increase by 64:

10.0.0.0

10.0.0.64

10.0.0.128

10.0.0.192

Complete Subnetting Plan

| Group | Network Address | Subnet Mask | Valid Host Range | Broadcast |
| --- | --- | --- | --- | --- |
| Delivery Partners | 10.0.0.0/26 | 255.255.255.192 | 10.0.0.1 – 10.0.0.62 | 10.0.0.63 |
| Restaurants | 10.0.0.64/26 | 255.255.255.192 | 10.0.0.65 – 10.0.0.126 | 10.0.0.127 |
| Customers | 10.0.0.128/26 | 255.255.255.192 | 10.0.0.129 – 10.0.0.190 | 10.0.0.191 |
| Admins | 10.0.0.192/26 | 255.255.255.192 | 10.0.0.193 – 10.0.0.254 | 10.0.0.255 |

Simple Diagram

10.0.0.0/24

│

┌─────────────┼─────────────┐

│             │             │

▼             ▼             ▼

/26 Subnet    /26 Subnet    /26 Subnet    /26 Subnet

│             │             │             │

▼             ▼             ▼             ▼

Delivery       Restaurants   Customers       Admins

Partners

Each group is logically separated from the others.

Note: Subnetting by itself does not automatically provide security isolation. In a real startup, VLANs, ACLs/firewall rules and appropriate authentication would normally be used as well.

4. College Wi-Fi: 172.16.0.0/16 and at Least 20 Subnets

Given

Network:

172.16.0.0/16

Required:

At least 20 subnets

Step 1 – Determine how many bits to borrow

Formula:

Number of subnets = 2ⁿ

We need at least 20.

Try different values:

2⁴ = 16       ← Not enough

2⁵ = 32       ← Enough

Therefore, we need to borrow 5 bits from the host portion.

Step 2 – Calculate the new prefix

Original prefix:

/16

Borrowed bits:

5

New prefix:

16 + 5 = /21

New subnet mask

/21

Binary:

11111111.11111111.11111000.00000000

Convert to decimal:

255.255.248.0

Therefore:

New Subnet Mask

255.255.248.0 (/21)

Step 3 – Calculate hosts per subnet

IPv4 has 32 total bits.

New prefix:

/21

Host bits:

32 - 21 = 11

Total addresses per subnet:

2¹¹ = 2048

Two addresses are reserved:

2048 - 2 = 2046

Therefore:

Usable hosts per subnet

2046 hosts

Step 4 – Number of subnets

Borrowed bits:

5

Number of subnets:

2⁵ = 32

Therefore, the network can be divided into 32 subnets, which is greater than the required 20.

Final Answer

| Item | Result |
| --- | --- |
| Original Network | 172.16.0.0/16 |
| Required Subnets | At least 20 |
| Bits Borrowed | 5 |
| Number of Subnets | 32 |
| New Prefix | /21 |
| New Subnet Mask | 255.255.248.0 |
| Host Bits | 11 |
| Total Addresses/Subnet | 2048 |
| Usable Hosts/Subnet | 2046 |

Example first few subnets

Because the third octet has a block size of:

256 - 248 = 8

The subnets begin:

172.16.0.0/21

172.16.8.0/21

172.16.16.0/21

172.16.24.0/21

172.16.32.0/21

...

There are 32 such /21 subnets across the original /16 network.

5. Why 192.168.1.300 Does Not Work

The IP address given is:

192.168.1.300

This is invalid.

Why?

An IPv4 address has four octets:

192 . 168 . 1 . 300

│     │    │     │

Octet Octet Octet Octet

Each octet must have a value between:

0 and 255

But the last octet is:

300

Since:

300 > 255

it cannot be a valid IPv4 address.

What should a valid address look like?

If the network is:

192.168.1.0/24

then:

Network address = 192.168.1.0

Broadcast       = 192.168.1.255

The valid host range is:

192.168.1.1

↓

192.168.1.254

Therefore, examples of valid host IP addresses are:

192.168.1.1

192.168.1.25

192.168.1.100

192.168.1.200

192.168.1.254

But:

192.168.1.300 ❌

is invalid.

Short message to your friend

Your IP address 192.168.1.300 is invalid because each IPv4 octet can only range from 0 to 255. If your network is 192.168.1.0/24, a valid host address would be between 192.168.1.1 and 192.168.1.254. For example, 192.168.1.100 is valid.

Quick Revision Table

| Question | Important Answer |
| --- | --- |
| Q1 | Classes A, B, C, D, E |
| Q2 | 192.168.10.45/24 → Network 192.168.10.0, Broadcast 192.168.10.255, Hosts .1–.254 |
| Q3 | 10.0.0.0/24 → four /26 subnets, 62 usable hosts each |
| Q4 | 172.16.0.0/16 → borrow 5 bits → /21, 255.255.248.0, 32 subnets, 2046 usable hosts/subnet |
| Q5 | 192.168.1.300 is invalid because an octet cannot exceed 255 |