IP routing and routing protocols Pending

Routing and OSPF – Detailed Assignment Answers

1. Network Diagram with Three Devices and IP Addresses

A simple home network can contain a router, a phone, a friend's laptop, and a smart TV.

Network Diagram

INTERNET

│

│ WAN

│

┌────────▼────────┐

│     ROUTER      │

│                 │

│ WAN: 203.0.113.2│

│ LAN: 192.168.1.1│

└────────┬────────┘

│

Wi-Fi / Ethernet

│

┌──────────────────┼──────────────────┐

│                  │                  │

▼                  ▼                  ▼

┌─────────────┐    ┌───────────────┐   ┌────────────┐

│   My Phone  │    │ Friend's      │   │ Smart TV   │

│             │    │ Laptop        │   │            │

│192.168.1.10 │    │192.168.1.20   │   │192.168.1.30│

└─────────────┘    └───────────────┘   └────────────┘

IP Address Table

| Device | IP Address | Subnet Mask | Default Gateway | Connection |
| --- | --- | --- | --- | --- |
| Router LAN | 192.168.1.1 | 255.255.255.0 | — | LAN/Wi-Fi |
| My Phone | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 | Wi-Fi |
| Friend's Laptop | 192.168.1.20 | 255.255.255.0 | 192.168.1.1 | Wi-Fi/Ethernet |
| Smart TV | 192.168.1.30 | 255.255.255.0 | 192.168.1.1 | Wi-Fi/Ethernet |
| Router WAN | 203.0.113.2 | Example only | ISP gateway | WAN |

Note: 203.0.113.0/24 is reserved for documentation/examples, so the WAN address above is not a real Internet address.

Explanation

All three devices are in the same network:

192.168.1.0/24

The router's LAN interface is:

192.168.1.1

Therefore, the devices use 192.168.1.1 as their default gateway when they need to communicate with a different network.

For example:

Phone → Router → Internet

For communication between two devices on the same subnet, the traffic can normally stay on the local network rather than being routed to another network.

2. Manual IP Routing Between Two Different Subnets

Scenario

Suppose you have a laptop on one subnet and your friend has a computer on another subnet.

Network

Laptop

192.168.10.10/24

│

│

▼

┌────────┐

│ Router1│

└───┬────┘

│

│ 10.0.0.0/30

│

▼

┌────────┐

│ Router2│

└───┬────┘

│

│

▼

Friend's PC

192.168.20.10/24

IP Addresses

| Device/Interface | IP Address |
| --- | --- |
| Laptop | 192.168.10.10/24 |
| Router 1 LAN | 192.168.10.1/24 |
| Router 1 → Router 2 | 10.0.0.1/30 |
| Router 2 → Router 1 | 10.0.0.2/30 |
| Router 2 LAN | 192.168.20.1/24 |
| Friend's PC | 192.168.20.10/24 |

Step 1 – Laptop creates the packet

Suppose your laptop wants to send data to:

Destination = 192.168.20.10

Your laptop checks its own network:

192.168.10.0/24

The destination is:

192.168.20.10

which is not in the same subnet.

Therefore, the laptop sends the packet to its default gateway:

192.168.10.1

Step 2 – Router 1 receives the packet

Router 1 receives the packet on its LAN interface.

Router 1 checks its routing table.

Example Router 1 routing table

| Destination Network | Subnet Mask | Next Hop | Interface |
| --- | --- | --- | --- |
| 192.168.10.0 | 255.255.255.0 | Directly connected | LAN |
| 10.0.0.0 | 255.255.255.252 | Directly connected | WAN |
| 192.168.20.0 | 255.255.255.0 | 10.0.0.2 | WAN |

Router 1 finds:

192.168.20.0/24

↓

Next hop = 10.0.0.2

So Router 1 forwards the packet to Router 2.

Step 3 – Packet travels to Router 2

The packet travels through the connection:

Router 1

10.0.0.1

│

│

▼

Router 2

10.0.0.2

Router 2 receives the packet.

It checks its routing table.

Example Router 2 routing table

| Destination Network | Subnet Mask | Next Hop | Interface |
| --- | --- | --- | --- |
| 192.168.20.0 | 255.255.255.0 | Directly connected | LAN |
| 10.0.0.0 | 255.255.255.252 | Directly connected | WAN |
| 192.168.10.0 | 255.255.255.0 | 10.0.0.1 | WAN |

Router 2 sees that:

192.168.20.0/24

is directly connected to its LAN interface.

Therefore, it sends the packet to:

192.168.20.10

Step 4 – Friend's computer receives the packet

The packet reaches the friend's PC:

192.168.20.10

The complete path is:

Laptop

192.168.10.10

↓

Router 1

192.168.10.1

↓

10.0.0.1 ───────── 10.0.0.2

↓

Router 2

↓

192.168.20.10

Friend's PC

How the routing decision is made

Routers look at the destination IP address and compare it with their routing table.

The most specific matching route is normally preferred.

For example:

Destination:

192.168.20.10

Matching route:

192.168.20.0/24

Router 1 therefore sends the packet toward the next hop:

10.0.0.2

Important distinction

The IP destination remains the final destination while the packet is being routed, but the Layer-2 Ethernet/MAC information is rewritten for each local link/hop.

3. Fictional Zomato-Style Delivery Network Routing Table

Suppose a food-delivery company has three branches:

- Ahmedabad

- Vadodara

- Surat

Each branch has its own subnet.

Network Diagram

┌──────────────┐

│   Main/Core  │

│    Router   │

└──────┬───────┘

│

┌─────────────┼─────────────┐

│             │             │

▼             ▼             ▼

Ahmedabad       Vadodara        Surat

10.10.1.0/24    10.10.2.0/24   10.10.3.0/24

Example Routing Table

| Destination Network | Subnet Mask | Next Hop | Interface | Branch |
| --- | --- | --- | --- | --- |
| 10.10.1.0/24 | 255.255.255.0 | Directly connected | G0/0 | Ahmedabad |
| 10.10.2.0/24 | 255.255.255.0 | 10.0.0.2 | G0/1 | Vadodara |
| 10.10.3.0/24 | 255.255.255.0 | 10.0.0.6 | G0/2 | Surat |

Example

Suppose a customer request needs to reach a server at:

10.10.3.50

The router checks the destination:

10.10.3.50

It matches:

10.10.3.0/24

The router therefore forwards it to:

Next Hop = 10.0.0.6

Interface = G0/2

The Surat branch router then delivers the packet to the server.

Example branch networks

Ahmedabad:

10.10.1.0/24

Hosts: 10.10.1.1 – 10.10.1.254

Vadodara:

10.10.2.0/24

Hosts: 10.10.2.1 – 10.10.2.254

Surat:

10.10.3.0/24

Hosts: 10.10.3.1 – 10.10.3.254

This type of design could separate branch traffic while allowing controlled communication between branches.

4. Static Routing vs Dynamic Routing

Two important routing approaches are static routing and dynamic routing.

Dynamic routing protocols include protocols such as RIP and OSPF.

Comparison Table

| Feature | Static Routing | Dynamic Routing |
| --- | --- | --- |
| Route creation | Manually configured | Learned automatically |
| Protocol examples | No routing protocol required | RIP, OSPF |
| Route changes | Administrator changes them | Routers can learn changes |
| Configuration | Simple for small networks | More complex |
| Scalability | Poor for large networks | Better for large networks |

Static Routing

In static routing, the network administrator manually enters routes.

Example:

Network:

10.20.0.0/24

Next hop:

192.168.1.2

A router might be configured with:

Destination: 10.20.0.0/24

Next Hop:    192.168.1.2

Advantages – Static Routing

1. Simple for small networks

If a company has only two or three routers, manually configured routes can be easy to manage.

Example: A small restaurant could have a simple network with one router and one backup network.

2. No routing-protocol overhead

The router does not need to exchange routing updates with other routers.

This reduces routing-protocol traffic and processing.

3. Greater administrative control

The administrator decides exactly which path traffic should take.

For example, an administrator could deliberately send administrative traffic through a particular router.

Disadvantages – Static Routing

1. Difficult to maintain in large networks

If a company has 100 routers, manually configuring routes on every router can become time-consuming.

2. Does not automatically adapt to failures

If a configured path fails, a static route does not automatically discover a better path.

The administrator may need to change the configuration.

3. Poor scalability

As the network grows, the number of routes that need manual configuration increases.

Dynamic Routing

Dynamic routing allows routers to exchange routing information and automatically learn routes.

Examples:

- RIP – Routing Information Protocol

- OSPF – Open Shortest Path First

Advantages – Dynamic Routing

1. Automatically learns routes

Routers can exchange information about available networks.

For example:

Router A ←→ Router B ←→ Router C

Router A can learn about networks connected to Router C.

2. Can adapt to network failures

Suppose the normal path is:

A → B → C

If the connection between B and C fails and another path exists:

A → D → C

a dynamic routing protocol can potentially select the alternate path.

3. Suitable for large networks

Dynamic routing is useful for organizations with many routers and network segments.

Food delivery example: A large delivery platform could have multiple regional data centers and network paths. Dynamic routing can help network infrastructure adapt when links become unavailable.

Disadvantages – Dynamic Routing

1. More complex

Protocols such as OSPF require additional configuration and understanding.

2. Uses network resources

Routers exchange routing information, which uses some bandwidth, CPU and memory.

3. Troubleshooting can be more difficult

Incorrect routing-protocol configuration can result in missing or incorrect routes, routing loops, or unexpected paths.

Static vs Dynamic – Real-World Example

Imagine a food-delivery company's infrastructure:

Main Data Center

│

┌──────────┴──────────┐

│                     │

Ahmedabad              Mumbai

Router                 Router

│                     │

Restaurants          Restaurants

With static routing, an administrator manually configures paths between the locations.

With dynamic routing, routers can exchange route information using OSPF and learn which networks are reachable through which paths.

For a small network, static routing may be perfectly adequate.

For a large, constantly changing infrastructure, dynamic routing is generally more practical.

Important: An app such as Zomato does not simply rely on one OSPF router. Large Internet services use many layers of networking, routing, load balancing, data centers and service infrastructure. OSPF is an example of routing used inside network infrastructures.

5. Sample OSPF Configuration Between Two Routers

Below is an AI-generated sample Cisco IOS configuration for a simple two-router OSPF lab.

Network Diagram

10.0.0.0/30

┌────────────────────────┐

│                        │

R1 G0/1                  G0/1 R2

10.0.0.1  ────────────  10.0.0.2

│                        │

G0/0                     G0/0

192.168.1.1               192.168.2.1

│                        │

▼                        ▼

LAN 1                    LAN 2

192.168.1.0/24           192.168.2.0/24

Router 1

enable

configure terminal

interface GigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

end

Router 2

enable

configure terminal

interface GigabitEthernet0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/1

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

router ospf 1

router-id 2.2.2.2

network 192.168.2.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

end

Important: This is a sample Cisco IOS lab configuration. Interface names and IOS syntax can vary by router/model/software version. Do not paste it into production equipment without adapting it to the actual interfaces and addressing.

Explanation of Router 1 Configuration

enable

enable

Enters privileged EXEC mode.

configure terminal

configure terminal

Enters global configuration mode.

Configure LAN interface

interface GigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

This selects Router 1's LAN interface and assigns:

IP = 192.168.1.1

Mask = 255.255.255.0

no shutdown activates the interface.

Configure connection between routers

interface GigabitEthernet0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

This gives Router 1 the address:

10.0.0.1/30

on the link connecting it to Router 2.

The /30 network provides four total addresses, with two normally usable host addresses:

10.0.0.0 = Network

10.0.0.1 = Router 1

10.0.0.2 = Router 2

10.0.0.3 = Broadcast

Start OSPF

router ospf 1

Creates or enters OSPF process 1.

The process ID is locally significant on the router.

Router ID

router-id 1.1.1.1

Gives Router 1 a unique OSPF router ID.

Router 2 uses:

router-id 2.2.2.2

Advertise LAN 1

network 192.168.1.0 0.0.0.255 area 0

This tells OSPF to enable OSPF on matching interfaces and advertise the 192.168.1.0/24 network in Area 0.

The wildcard mask:

0.0.0.255

corresponds to a /24 network.

Advertise the router-to-router network

network 10.0.0.0 0.0.0.3 area 0

This enables OSPF on the 10.0.0.0/30 connection and allows the two routers to form an OSPF neighbor relationship over that link.

Router 2

Router 2 is configured similarly.

Its LAN is:

192.168.2.0/24

and its router-to-router interface is:

10.0.0.2/30

It advertises both networks through OSPF.

How OSPF Works in This Example

Initially:

R1 knows:

192.168.1.0/24

10.0.0.0/30

and:

R2 knows:

192.168.2.0/24

10.0.0.0/30

After the OSPF neighbors establish communication, they exchange routing information.

R1 learns:

192.168.2.0/24

through R2.

R2 learns:

192.168.1.0/24

through R1.

Therefore, a device on LAN 1 can communicate with a device on LAN 2.

PC on LAN 1

192.168.1.10

│

▼

R1

10.0.0.1

│

│ OSPF-learned route

▼

R2

10.0.0.2

│

▼

192.168.2.10

PC on LAN 2

Useful OSPF verification commands

On a Cisco router, you can check the configuration with:

show ip interface brief

Shows interface status and IP addresses.

show ip route

Shows the routing table.

show ip ospf neighbor

Shows OSPF neighbor relationships.

show ip protocols

Shows routing protocol information.

An OSPF-learned route in the routing table is normally marked with O.

Final Summary

| Task | Main Result |
| --- | --- |
| 1. Network diagram | Phone, laptop and TV use the router as their gateway on 192.168.1.0/24 |
| 2. Manual routing | Packet travels from Laptop → Router 1 → Router 2 → Friend's PC based on routing-table matches |
| 3. Zomato-style network | Ahmedabad, Vadodara and Surat use separate subnets with specified next hops/interfaces |
| 4. Static vs Dynamic | Static is simple and controlled; dynamic such as OSPF/RIP is more scalable and can adapt to topology changes |
| 5. OSPF | Two routers exchange route information so each can learn the other's LAN automatically |

Key exam point: A router makes a forwarding decision primarily by examining the destination IP address and selecting the best matching route in its routing table. Static routes are manually configured, while dynamic protocols such as OSPF allow routers to learn and update routes automatically.