Network Cable and connectors Pending

Network Cable & RJ-45 Practical – Detailed Answers

1. Cat6 Ethernet Cable – T568B Wiring Diagram

A Cat6 Ethernet cable contains four twisted pairs, making a total of 8 individual wires. For a standard straight-through Ethernet cable, the T568B color arrangement is commonly used.

4

T568B pin order

When looking at the RJ-45 plug with the contacts facing upward and the cable entering from the rear, the wires are arranged from Pin 1 to Pin 8 as follows:

RJ-45 CONNECTOR – T568B

┌─────────────────────────┐

│ 1  2  3  4  5  6  7  8 │

│ │  │  │  │  │  │  │  │ │

│ 🟧 🟧 🟩 🟦 🟦 🟩 🟫 🟫 │

└─────────────────────────┘

│

│ Cat6 Cable

▼

Pin 1  → White/Orange

Pin 2  → Orange

Pin 3  → White/Green

Pin 4  → Blue

Pin 5  → White/Blue

Pin 6  → Green

Pin 7  → White/Brown

Pin 8  → Brown

T568B color table

| Pin | Wire Color | Pair |
| --- | --- | --- |
| 1 | White/Orange | Orange pair |
| 2 | Orange | Orange pair |
| 3 | White/Green | Green pair |
| 4 | Blue | Blue pair |
| 5 | White/Blue | Blue pair |
| 6 | Green | Green pair |
| 7 | White/Brown | Brown pair |
| 8 | Brown | Brown pair |

The T568B sequence is White-Orange, Orange, White-Green, Blue, White-Blue, Green, White-Brown, Brown.

Important points

- An Ethernet cable has 4 twisted pairs.

- Each pair consists of one solid-colored wire and one white wire with a colored stripe.

- Keep the wires in the correct order before inserting them into the RJ-45 connector.

- For a normal straight-through cable, use the same wiring standard on both ends, e.g. T568B on both ends.

- Cat6 commonly uses an RJ-45/8P8C connector for copper Ethernet connections.

2. Three Types of Network Cables and Their Uses

A. Cat5e Ethernet Cable

Type: Copper twisted-pair cable
Connector: RJ-45

Example: Wi-Fi Router

A Cat5e cable can connect a Wi-Fi router to a computer, switch, or other Ethernet device.

Why: Cat5e supports Gigabit Ethernet over appropriate links up to the standard 100 m channel length, making it suitable for normal home and small-office networking.

Internet/ONT

│

│ Cat5e

▼

Wi-Fi Router

│

├──── Laptop

└──── Desktop PC

B. Cat6 Ethernet Cable

Type: Copper twisted-pair cable
Connector: RJ-45

Example: Gaming PC / POS Terminal

A Cat6 cable can connect a gaming PC, POS terminal, network switch, or workstation to a network switch.

Why: Cat6 provides higher performance than older categories and can support Gigabit Ethernet over standard 100 m links; suitable equipment can use higher speeds over Cat6 depending on distance and hardware.

Example:

Gaming PC

│

│ Cat6

▼

Network Switch

│

├── Gaming PC

├── POS Terminal

└── Router

C. Fiber-Optic Cable

Type: Optical cable using light
Connector: Depends on equipment, commonly LC or other fiber connectors

Example: IPL Streaming Server / Data-Center Network

Fiber can be used to connect high-speed switches, servers, or data-center network equipment.

Why: Fiber supports high bandwidth, long-distance connections, and is resistant to electromagnetic interference. It is especially useful for network links longer than the normal 100 m copper Ethernet channel or for high-performance backbone connections.

Streaming Server

│

│ Fiber Optic

▼

High-Speed Switch

│

▼

Internet / Data Center

Comparison

| Cable | Medium | Common Connector | Example | Main Reason |
| --- | --- | --- | --- | --- |
| Cat5e | Copper | RJ-45 | Wi-Fi router → PC | Affordable and suitable for Gigabit Ethernet |
| Cat6 | Copper | RJ-45 | Gaming PC / POS | Higher-performance copper Ethernet |
| Fiber optic | Light through fiber | LC/SFP-based interfaces, depending on equipment | Streaming server / data-center link | Long distance, high bandwidth, EMI immunity |

Cisco notes that four-pair copper Ethernet is generally limited to a 100 m channel, while fiber is a common choice for longer links and electrically noisy environments.

3. RJ-45 Crimping Process

For this task, a suitable YouTube demonstration is “How to Crimp RJ45 Connector”, which demonstrates Cat6 cable preparation, T568B wire arrangement, connector insertion, crimping, and testing.

Tools required

- Cat5e/Cat6 Ethernet cable

- RJ-45 connectors

- RJ-45 crimping tool

- Cable stripper/cutter

- Cable tester

- Optional: cable boots

Step 1 – Measure and cut the cable

Cut the Ethernet cable to the required length.

Leave a little extra length so that the connector can be prepared comfortably.

Step 2 – Remove the outer jacket

Use the stripper on the cable to carefully remove a short section of the outer plastic jacket.

Before:

===============================

Cat6 cable

After:

====[ exposed twisted pairs ]==

Important: Do not cut or damage the internal wires.

Step 3 – Separate the four twisted pairs

You will find four pairs:

- Orange

- Green

- Blue

- Brown

Carefully separate them.

Step 4 – Arrange the wires according to T568B

Arrange all eight wires in this exact order:

1. White/Orange

2. Orange

3. White/Green

4. Blue

5. White/Blue

6. Green

7. White/Brown

8. Brown

Do not change the order after arranging them.

Step 5 – Straighten the wires

Straighten the wires gently using your fingers.

Avoid removing too much twisting from the pairs because excessive untwisting can negatively affect signal performance.

Step 6 – Cut the wires evenly

Trim the eight wires so their ends are equal in length.

White/Orange ─┐

Orange       ─┤

White/Green  ─┤

Blue         ─┤

White/Blue   ─┤── Equal length

Green        ─┤

White/Brown  ─┤

Brown        ─┘

Step 7 – Insert the wires into the RJ-45 connector

Carefully push all eight wires into the connector.

Make sure:

- Every wire goes into its own channel.

- The order remains T568B.

- The wires reach the front of the connector.

- The cable jacket enters far enough into the connector so the connector's strain relief can grip it.

Step 8 – Check the pin order

Before crimping, look through the transparent connector.

Verify:

1 → White/Orange

2 → Orange

3 → White/Green

4 → Blue

5 → White/Blue

6 → Green

7 → White/Brown

8 → Brown

This is the last easy opportunity to correct the wiring.

Step 9 – Crimp the connector

Insert the RJ-45 connector into the crimping tool.

Squeeze the tool firmly so the metal contacts are pressed onto the wires and the connector locks onto the cable.

Step 10 – Make the other end

For a normal straight-through Ethernet cable, use T568B on the second end as well.

T568B                       T568B

RJ-45                       RJ-45

┌───────┐                   ┌───────┐

│1 2 3 4│===================│1 2 3 4│

│5 6 7 8│      Cat6         │5 6 7 8│

└───────┘                   └───────┘

Step 11 – Test the cable

Connect both ends to a cable tester.

A correctly wired cable should show the expected sequence:

1 → 1

2 → 2

3 → 3

4 → 4

5 → 5

6 → 6

7 → 7

8 → 8

A cable tester can identify incorrectly connected or missing wires.

Two common mistakes to avoid

Mistake 1 – Wrong color order:
Putting the wires in the wrong T568B sequence can cause the cable to fail or operate incorrectly.

Mistake 2 – Wires not fully inserted:
If the eight wires do not reach the correct positions inside the RJ-45 connector before crimping, some pins may not make proper contact.

Other common mistakes include excessive untwisting and accidentally using different wiring standards at the two ends.

4. Cable Selection for a Gaming Zone During an IPL Screening Event

Scenario

A gaming zone needs to connect:

- Multiple gaming consoles

- Gaming PCs

- Large screens/TVs

- Network switches

- Router/Internet connection

The network should provide high speed, low latency, reliability, and stable connections.

Recommended cable: Cat6 Ethernet

For the individual gaming consoles/PCs and nearby screens or network devices with Ethernet ports, I would use Cat6 Ethernet cable with RJ-45 connectors.

Why Cat6?

1. High speed

Cat6 is suitable for Gigabit Ethernet and can support higher speeds with compatible equipment over appropriate distances.

2. Low latency

A wired Ethernet connection generally provides a more predictable connection than relying on Wi-Fi in a crowded gaming area.

3. Reliability

A wired connection is less affected by wireless congestion and interference than Wi-Fi.

4. Suitable for gaming

Gaming requires stable communication with low delay. Cat6 connected directly to a switch gives each console/PC a dedicated switched Ethernet link.

Recommended design

Internet

│

│

Router

│

Cat6

│

┌──────────┐

│  Switch  │

└──────────┘

/   |   \

/    |    \

Cat6   Cat6   Cat6

/      |      \

Console  Console  Gaming PC

│

Cat6

│

Smart TV / Display

For a larger gaming zone

If the gaming zone is spread across a large venue, I would use fiber optic cable for the high-speed backbone/uplink between network switches and use Cat6 for the shorter connections to individual devices.

Router

│

Fiber

│

Core Switch

/       \

Fiber       Fiber

/           \

Switch A       Switch B

/ | \           / | \

Cat6 Cat6 Cat6  Cat6 Cat6 Cat6

│    │    │      │    │    │

PC   Console TV    PC Console TV

Fiber is particularly useful when the distance exceeds the normal 100 m copper Ethernet channel or when strong electromagnetic interference is a concern.

Connector choices

| Connection | Recommended cable | Connector/interface |
| --- | --- | --- |
| Gaming PC → Switch | Cat6 | RJ-45 |
| Console → Switch | Cat6 | RJ-45 |
| Router → Switch | Cat6 | RJ-45 |
| Switch → Switch, short distance | Cat6/Cat6A as supported | RJ-45 |
| Switch → Switch, long/high-speed backbone | Fiber | Compatible SFP/SFP+ and fiber connector |
| TV/display → Console | HDMI, not Ethernet | HDMI |

Important: Ethernet is primarily for network connectivity. A TV's video connection from a console is normally HDMI, while Cat6 connects the console/PC to the network.

Final choice

For a normal gaming zone:

Cat6 + RJ-45 = best choice for device-to-switch network connections.

For long/high-speed backbone links:

Fiber optic + compatible optical transceivers = better choice.

5. Network Cable/Connector Photo and Description

For this assignment, you can use a photo from your home, college, café, or an online source. Since I cannot take a photo of your actual home or college setup, you should attach your own photo if possible.

Example photo

5

Example description for your assignment

Photo: Cat6 Ethernet cable connected to a Wi-Fi router/network switch.

Cable type: Cat6 Ethernet cable

Connector: RJ-45

Devices connected: The cable connects the router/switch to a computer, access point, or another network device.

Purpose: It provides a wired Ethernet connection for transferring network data between the devices. Compared with a wireless connection, a wired Ethernet link can provide a stable and predictable connection for suitable network equipment.

You can write this below your actual photo

Observation: The photograph shows a blue Cat6 Ethernet cable with RJ-45 connectors. One end is connected to the LAN/Ethernet port of a network device, while the other end is connected to a computer/switch. The cable is used to provide a wired network connection for data communication.

Note: If your actual cable says Cat5e, write Cat5e instead of Cat6. Check the printing on the cable jacket before submitting the assignment.

Quick Revision

| Topic | Answer |
| --- | --- |
| T568B Pin 1 | White/Orange |
| T568B Pin 2 | Orange |
| T568B Pin 3 | White/Green |
| T568B Pin 4 | Blue |
| T568B Pin 5 | White/Blue |
| T568B Pin 6 | Green |
| T568B Pin 7 | White/Brown |
| T568B Pin 8 | Brown |
| Common copper connector | RJ-45 |
| Recommended gaming-zone cable | Cat6 |
| Long/high-speed backbone | Fiber optic |
| Cable tester purpose | Checks wire/pin continuity and order |
| Straight-through cable | Same standard on both ends, e.g. T568B–T568B |