OSI LayerPending

OSI 7-Layer Model – Detailed Answers

1. OSI 7-Layer Model Diagram

OSI stands for Open Systems Interconnection. It is a seven-layer model used to understand how data travels between devices over a network.

OSI 7-Layer Diagram

┌─────────────────────────────────────────────┐

│  Layer 7 – APPLICATION                      │

│  HTTP, HTTPS, DNS, FTP, SMTP                │

│  Example: Web browser / web server         │

├─────────────────────────────────────────────┤

│  Layer 6 – PRESENTATION                     │

│  Encryption, Compression, Data Formatting   │

│  Example: TLS/SSL, JPEG, encryption         │

├─────────────────────────────────────────────┤

│  Layer 5 – SESSION                           │

│  Establishes and manages communication      │

│  Example: Session management                │

├─────────────────────────────────────────────┤

│  Layer 4 – TRANSPORT                         │

│  TCP, UDP                                   │

│  Example: TCP communication                 │

├─────────────────────────────────────────────┤

│  Layer 3 – NETWORK                           │

│  IP, Routing                                │

│  Example: Router                            │

├─────────────────────────────────────────────┤

│  Layer 2 – DATA LINK                        │

│  Ethernet, MAC, Frames                      │

│  Example: Switch / Ethernet NIC             │

├─────────────────────────────────────────────┤

│  Layer 1 – PHYSICAL                         │

│  Bits, electrical/radio/optical signals     │

│  Example: Cable, fiber, radio, repeater      │

└─────────────────────────────────────────────┘

Layers with examples

| Layer | Name | Main Function | Example |
| --- | --- | --- | --- |
| 7 | Application | Provides network services to applications | HTTP, HTTPS, DNS, browser |
| 6 | Presentation | Data formatting, encryption and compression | TLS/SSL, JPEG |
| 5 | Session | Establishes, manages and terminates sessions | Session management |
| 4 | Transport | End-to-end delivery, reliability and flow control | TCP, UDP |
| 3 | Network | Logical addressing and routing | IP, Router |
| 2 | Data Link | Frames, MAC addressing and error detection | Ethernet, Switch |
| 1 | Physical | Transmits raw bits as signals | Ethernet cable, fiber, radio |

Easy way to remember

From Layer 7 to Layer 1:

All
People
Seem
To
Need
Data
Processing

- A → Application

- P → Presentation

- S → Session

- T → Transport

- N → Network

- D → Data Link

- P → Physical

2. Match the Networking Scenarios with OSI Layers

a) Encrypting data before sending

Scenario: Encrypting data before sending: Presentation Layer (Layer 6)

Explanation: The Presentation layer is responsible for data representation, encryption/decryption and compression. For example, encryption associated with TLS helps protect data exchanged between applications.

b) Routing packets across networks

Scenario: Routing packets across networks: Network Layer (Layer 3)

Explanation: The Network layer uses logical addresses such as IP addresses and determines how packets should travel between different networks. Routers primarily operate at this layer.

c) Converting data to electrical signals

Scenario: Converting data to electrical signals: Physical Layer (Layer 1)

Explanation: The Physical layer transmits raw bits using electrical, optical or radio signals through cables, fiber or wireless media.

d) Opening a web page in your browser

Scenario: Opening a web page in your browser: Application Layer (Layer 7)

Explanation: Web browsers use application-layer protocols such as HTTP/HTTPS to request web pages from web servers.

e) Detecting and correcting errors in frames

Scenario: Detecting and correcting errors in frames: Data Link Layer (Layer 2)

Explanation: The Data Link layer handles frames and provides error detection mechanisms such as the Ethernet Frame Check Sequence (FCS). Some data-link technologies can also provide retransmission or correction mechanisms.

Final answers

| Scenario | Correct Layer |
| --- | --- |
| a) Encrypting data before sending | Presentation – Layer 6 |
| b) Routing packets across networks | Network – Layer 3 |
| c) Converting data to electrical signals | Physical – Layer 1 |
| d) Opening a web page in a browser | Application – Layer 7 |
| e) Detecting errors in frames | Data Link – Layer 2 |

3. OSI Layers Involved When Sending a WhatsApp Message

Suppose I send "Hello" to a friend using WhatsApp. Multiple OSI layers work together to deliver the message.

Layer 7 – Application

The WhatsApp application provides the user interface for composing and sending the message.

- The user types "Hello".

- The application prepares the message for transmission.

- Application-level communication uses protocols and services provided by the app and its network infrastructure.

Example: WhatsApp application.

Layer 6 – Presentation

The data needs to be represented and protected before transmission.

- Data can be encoded into a suitable format.

- Encryption helps protect the communication.

- At the receiving side, protected/encoded data is processed so the application can use it.

Example: Encryption and data representation.

Layer 4 – Transport

The Transport layer provides end-to-end communication between applications.

- TCP or UDP are common transport-layer protocols.

- It uses port numbers to identify applications/services.

- Depending on the communication system, reliability and ordering mechanisms may be provided here or by higher layers.

Example: TCP/UDP.

Layer 3 – Network

The Network layer is responsible for moving packets between different networks.

- The phone has an IP address.

- Packets contain source and destination IP information.

- Routers forward packets toward their destination.

Example: IP and routers.

Layer 2 – Data Link

The Data Link layer handles communication over the local network connection.

If the phone is using Wi-Fi:

- The phone communicates with the Wi-Fi access point/router.

- Frames use local network addressing such as MAC addresses.

- The wireless network handles transmission over the local link.

Example: Wi-Fi/Ethernet and MAC addressing.

Layer 1 – Physical

Finally, the actual bits are transmitted as physical signals.

For Wi-Fi:

Smartphone

↓

Radio signals

↓

Wi-Fi Router

For wired Ethernet:

Computer

↓

Electrical signals

↓

Ethernet cable

↓

Switch

Simplified WhatsApp data flow

User types "Hello"

↓

Layer 7 – Application

↓

Layer 6 – Presentation

Encryption / Formatting

↓

Layer 4 – Transport

TCP/UDP

↓

Layer 3 – Network

IP + Routing

↓

Layer 2 – Data Link

Wi-Fi / Ethernet frames

↓

Layer 1 – Physical

Radio / Electrical / Optical signals

↓

Network

↓

Friend's device

↓

Layers are processed upward

↓

WhatsApp displays "Hello"

Conclusion: Although the user sees only the WhatsApp application, multiple networking layers work together to move the message from one device to another.

4. Website Cannot Be Opened but Its IP Address Can Be Pinged

Scenario

Suppose a website cannot be opened in a browser, but you can successfully run:

ping <website-IP-address>

and receive replies.

What does this tell us?

A successful ping generally indicates that:

- The computer has network connectivity.

- The IP address is reachable using ICMP.

- Basic IP routing is working.

- Lower network layers are likely functioning.

Therefore, the problem is probably above Layer 3, although there are important exceptions.

Possible problem at Layer 7 – Application

The browser uses application-layer protocols such as HTTP or HTTPS.

Possible problems include:

- Web server/application service is down.

- The website is not responding to HTTP/HTTPS requests.

- The server is configured incorrectly.

- The browser has an issue.

- The requested website/application is unavailable.

Example

Ping IP → SUCCESS

HTTP/HTTPS → FAIL

This suggests that the server is reachable at the IP level, but the web service may not be working.

Possible problem at Layer 6 – Presentation

HTTPS uses encryption and certificate mechanisms.

Possible problems include:

- Invalid or expired certificate.

- TLS/SSL negotiation failure.

- Unsupported TLS configuration.

- Encryption-related browser/server problems.

For example, the browser might display a certificate or secure-connection error even though the server's IP responds to ping.

Possible problem at Layer 4 – Transport

The browser normally needs a TCP connection for traditional HTTP/HTTPS traffic.

Common ports include:

- HTTP → TCP port 80

- HTTPS → TCP port 443

A firewall or server configuration could block the required TCP port even though ICMP ping works.

Ping / ICMP → SUCCESS

TCP 443 → BLOCKED

HTTPS → FAIL

This is a common reason why a host can respond to ping while a website remains inaccessible.

Could it be DNS?

Yes, if you normally access the site by its domain name rather than directly by IP address.

For example:

www.example.com → DNS lookup fails

but:

ping 93.184.216.34 → SUCCESS

In that case, the problem may involve DNS, which is an Application-layer (Layer 7) service.

You could test with:

nslookup www.example.com

Troubleshooting steps

Step 1 – Test IP connectivity

ping <website-IP>

If successful, basic IP connectivity exists.

Step 2 – Test DNS

nslookup website.com

If DNS fails, investigate the DNS configuration/server.

Step 3 – Test the web service

For HTTPS, check whether TCP port 443 is reachable using an appropriate network diagnostic tool.

Step 4 – Check browser

- Try another browser.

- Clear browser cache if appropriate.

- Check proxy settings.

- Check whether extensions are interfering.

Step 5 – Check firewall/security

A firewall may allow ICMP ping but block TCP port 80 or 443.

Step 6 – Check the server

If the network connection and DNS are working but HTTP/HTTPS still fails, the website's web server or application may be down or misconfigured.

Final conclusion

If a website's IP address responds to ping but the website does not open, the basic Network layer connectivity is probably working. The most likely areas to investigate are Layer 4 (Transport) for blocked TCP ports, Layer 6 (Presentation) for TLS/HTTPS problems, and Layer 7 (Application) for DNS, HTTP/HTTPS, browser, or web-server problems.

Important: Ping uses ICMP, not HTTP/HTTPS. Therefore, a successful ping does not prove that the website's web service is working.