Review and Practice Practical

Cisco Packet Tracer Practical: Home Wi-Fi Network

Aim: To create a home network in Cisco Packet Tracer using three PCs and one wireless router, configure static IPv4 addresses, and test connectivity using the ping command.

1. Create a new Packet Tracer project

Telecommunication Engineering

Step 1: Open Cisco Packet Tracer

- Launch Cisco Packet Tracer on your computer.

- Select File → New.

- Save the project using File → Save As.

- Enter the filename HomeWiFiNetwork.pkt.

- Choose your desired folder and save the project.

Step 2: Add three PCs

- Select End Devices from the bottom-left device panel.

- Drag three PCs into the workspace.

- Rename them PC0, PC1, and PC2 (or PC1, PC2, and PC3 if your instructor prefers numbering from 1).

Step 3: Add a wireless router

- Open Network Devices and look under Wireless Devices.

- Drag a compatible wireless router into the workspace.

- Rename it Home-Wireless-Router.

The exact router model and menu names may vary by Packet Tracer version.

2. Connect the PCs to the wireless router

For the simplest reliable setup, connect all three PCs to the router using Ethernet cables. This still creates a home network through a wireless router, which typically includes built-in LAN switch ports and Wi-Fi.

Wireless Network pada Cisco Packet Tracer! | by Zulfa Nashihin | Medium

Home Wireless Router

Router LAN IP: 192.168.10.1

Copper straight-through cables to LAN ports

PC0

192.168.10.2

PC1

192.168.10.3

PC2

192.168.10.4

Steps to connect:

- Click Connections (the lightning-bolt icon).

- Select Copper Straight-Through.

- Click PC0 and choose FastEthernet0.

- Click the wireless router and choose a LAN port, such as Ethernet 1.

- Repeat for PC1 and PC2 using separate LAN ports.

- Wait for the link indicators to turn green.

Important: Do not connect the PCs to the router's Internet/WAN port for this setup. Use its LAN ports. If your selected router model has no Ethernet LAN ports, use a compatible wireless router model or connect an Ethernet switch to its LAN interface.

3. Assign static IP addresses to the PCs

We will use the network 192.168.10.0/24. All PCs and the router's LAN interface must belong to the same subnet.

IP addressing table

| Device | IPv4 address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| Router LAN | 192.168.10.1 | 255.255.255.0 | Not required |
| PC0 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| PC1 | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 |
| PC2 | 192.168.10.4 | 255.255.255.0 | 192.168.10.1 |

Configure PC0

IP Static menggunakan Cisco Packet Tracer | by Munawirsyah | Medium

Step 1: Click PC0 in the workspace.

Step 2: Open the Desktop tab.

Step 3: Click IP Configuration.

Step 4: Select Static and enter:

- IPv4 Address: 192.168.10.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.10.1

Configure PC1

Click PC1 → Desktop → IP Configuration → Static, then enter:

- IPv4 Address: 192.168.10.3

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.10.1

Configure PC2

Click PC2 → Desktop → IP Configuration → Static, then enter:

- IPv4 Address: 192.168.10.4

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.10.1

Configure the router's LAN IP

The router's LAN address must be 192.168.10.1 for the gateway settings above to work.

For a Linksys-style wireless router in Packet Tracer:

- Click the wireless router.

- Open GUI.

- Find Setup → Basic Setup or the equivalent LAN/network settings.

- Set the router's local IP address to 192.168.10.1 and subnet mask to 255.255.255.0.

- Save the settings.

If your router model uses a different interface, open its Config tab and look for the LAN or Ethernet interface. Ensure the interface is configured with the same address. Avoid duplicate IP addresses, and keep the PCs' addresses outside the DHCP pool if DHCP is enabled.

4. Test connectivity using the ping command

The ping command checks whether another device can be reached using IP networking. Test each PC against the router and the other PCs.

Test from PC0

- Click PC0.

- Open Desktop → Command Prompt.

- Enter these commands one at a time:

ping 192.168.10.1

ping 192.168.10.3

ping 192.168.10.4

The first command tests the router, the second tests PC1, and the third tests PC2.

Test from PC1

Open PC1 → Desktop → Command Prompt, then run:

ping 192.168.10.1

ping 192.168.10.2

ping 192.168.10.4

Test from PC2

Open PC2 → Desktop → Command Prompt, then run:

ping 192.168.10.1

ping 192.168.10.2

ping 192.168.10.3

Expected output

A successful test typically shows output similar to:

Reply from 192.168.10.1: bytes=32 time<1ms TTL=...

The exact time and TTL may vary. The first ping may time out while ARP resolves a device's MAC address, so repeat the command before diagnosing a failure.

| Test | Expected result |
| --- | --- |
| PC0 → Router | Successful replies |
| PC0 → PC1 | Successful replies |
| PC0 → PC2 | Successful replies |
| PC1 → PC2 | Successful replies |

If ping fails, check the IP address, subnet mask, gateway, cable connections, router LAN address, and link status. In Packet Tracer, you can also use Add Simple PDU (the envelope icon) to test connectivity visually.

5. Prepare the Word or PDF report with screenshots

Your assignment requires evidence from your own Packet Tracer project. Take these screenshots after completing the configuration.

Screenshot 1 — Network topology

Capture the complete Packet Tracer workspace showing the wireless router, PC0, PC1, PC2, and all three connections.

Suggested caption: Figure 1. Home Wi-Fi network topology.

Screenshot 2 — PC0 IP configuration

Capture PC0's IP Configuration window showing 192.168.10.2, subnet mask 255.255.255.0, and gateway 192.168.10.1.

Screenshot 3 — PC1 IP configuration

Capture PC1's IP Configuration window showing 192.168.10.3, the subnet mask, and the gateway.

Screenshot 4 — PC2 IP configuration

Capture PC2's IP Configuration window showing 192.168.10.4, the subnet mask, and the gateway.

Screenshot 5 — Ping test results

Capture a PC's Command Prompt showing successful ping replies from the router and the other two PCs.

How to insert the screenshots in Microsoft Word

- Open Microsoft Word and create a blank document.

- Add the title Home Wi-Fi Network Using Cisco Packet Tracer.

- Add your name, roll number, class, and submission date.

- Copy the aim, topology, IP addressing table, configuration steps, and ping tests from this answer into the report.

- For each screenshot, press Win + Shift + S, select the required area, then paste it into Word using Ctrl + V.

- Add a caption below each screenshot.

- Save the document as HomeWiFiNetwork_Report.docx.

- To make a PDF, select File → Save As or Export → Create PDF and save it as HomeWiFiNetwork_Report.pdf.

Remember to save the Packet Tracer project separately as HomeWiFiNetwork.pkt. The Word/PDF report documents the work; the .pkt file contains the editable network project.

6. Result and conclusion

Result: A home network was designed in Cisco Packet Tracer using one wireless router and three PCs. Static IPv4 addresses were assigned in the 192.168.10.0/24 subnet, with the router's LAN address configured as the default gateway. Connectivity was tested using ping.

Conclusion: Devices in the same subnet can communicate through the router's LAN switch ports without requiring inter-subnet routing. The ping tests help verify correct IP addressing and connectivity.

For your practical submission, record the actual ping results and include screenshots captured from your own Packet Tracer setup rather than illustrative images.