Switch and Router Configuration Basics Theory

Cisco Packet Tracer Practical: VLANs, Static Routing and Inter-VLAN Communication

1. Create VLAN 10 (Sales) and VLAN 20 (Support)

Aim: Create two VLANs on a Cisco switch to separate Sales and Support devices logically.

Step 1: Create the topology

In Cisco Packet Tracer, add:

- 1 switch (e.g., Cisco 2960)

- 5 PCs (3 Sales PCs and 2 Support PCs)

Connect the PCs to the switch using Copper Straight-Through cables.

VLAN topology

Example layout: three Sales PCs and two Support PCs connected to one switch.

Step 2: Configure the switch

Click the switch, open the CLI tab, and enter the following commands:

enable

configure terminal

vlan 10

name Sales

exit

vlan 20

name Support

exit

The two VLANs have now been created.

2. Assign switch ports and verify VLAN membership

We will assign ports Fa0/1–Fa0/3 to Sales and Fa0/4–Fa0/5 to Support.

Step 1: Assign ports to VLAN 10

configure terminal

interface range fastEthernet 0/1-3

switchport mode access

switchport access vlan 10

exit

Step 2: Assign ports to VLAN 20

interface range fastEthernet 0/4-5

switchport mode access

switchport access vlan 20

exit

Step 3: Save and verify

end

show vlan brief

Expected output format:

| VLAN | Name | Ports |
| --- | --- | --- |
| 1 | default | Remaining unassigned ports |
| 10 | Sales | Fa0/1, Fa0/2, Fa0/3 |
| 20 | Support | Fa0/4, Fa0/5 |

Save the configuration:

copy running-config startup-config

Your switch ports should now be assigned to their respective VLANs.

3. Connect two routers and configure a static route

Aim: Connect Router 1 to Router 2 and configure Router 1 to reach the LAN behind Router 2.

Network addressing plan

Router-to-router topology

Step 1: Configure Router 1

Assume Router 1 uses GigabitEthernet0/0 for LAN 1 and GigabitEthernet0/1 for the router link.

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

Step 2: Configure Router 2

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

Connect the routers using an Ethernet cable and use the interface names available on your router model.

Step 3: Configure a static route on Router 1

configure terminal

ip route 192.168.2.0 255.255.255.0 10.0.0.2

end

Explanation:

- 192.168.2.0 is the destination LAN.

- 255.255.255.0 is its subnet mask.

- 10.0.0.2 is Router 2's next-hop IP address.

For two-way communication, configure a return route on Router 2 as well:

configure terminal

ip route 192.168.1.0 255.255.255.0 10.0.0.1

end

Verify the route and interface status:

show ip route

show ip interface brief

4. Why does ping fail between Sales and Support VLANs?

Scenario: A PC in Sales (VLAN 10) attempts to ping a PC in Support (VLAN 20).

Example:

- Sales PC: 192.168.10.2/24

- Support PC: 192.168.20.2/24

If the switch only has VLANs and access ports configured, the ping will generally fail because the two VLANs are separate Layer 2 broadcast domains. A standard Layer 2 switch does not route traffic between them.

Solution A: Router-on-a-stick

Connect a router to the switch using a trunk port, then configure router subinterfaces for VLAN 10 and VLAN 20.

Example switch configuration for a router connected to Fa0/24:

configure terminal

interface fastEthernet 0/24

switchport mode trunk

end

Example router configuration, assuming the router connects through GigabitEthernet0/0:

configure terminal

interface gigabitEthernet 0/0

no shutdown

exit

interface gigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

interface gigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

end

The Sales PC should use 192.168.10.1 as its default gateway, and the Support PC should use 192.168.20.1.

Solution B: Layer 3 switch

A Layer 3 switch can route between VLANs using switched virtual interfaces (SVIs), with IP routing enabled. This is another way to allow communication between VLAN 10 and VLAN 20.

Conclusion: VLANs separate devices into logical networks. To enable communication between different VLANs, configure inter-VLAN routing using router-on-a-stick or a Layer 3 switch.