Switching and VLANS Pending

VLAN Configuration and Testing – Detailed Assignment

1. Network Diagram with VLANs

A VLAN (Virtual Local Area Network) divides one physical switch into multiple logical networks.

For this example, we will use:

- VLAN 10 – Streaming

- VLAN 20 – Personal Devices

Network Diagram

┌───────────────────┐

│      SWITCH       │

│                   │

│ VLAN 10: Streaming│

│ VLAN 20: Personal │

└─────┬───┬───┬─────┘

│   │   │

┌─────────────┘   │   └──────────────┐

│                 │                  │

▼                 ▼                  ▼

┌────────────┐    ┌────────────┐     ┌────────────┐

│   Laptop   │    │ Smart TV   │     │   Phone    │

│  VLAN 10   │    │  VLAN 10   │     │  VLAN 20   │

│ Streaming  │    │ Streaming  │     │ Personal   │

└────────────┘    └────────────┘     └────────────┘

We can also include a friend's phone:

┌─────────────────┐

│     SWITCH      │

│                 │

│ VLAN 10         │

│ Streaming       │

│                 │

│ VLAN 20         │

│ Personal        │

└─┬────┬────┬────┬┘

│    │    │    │

│    │    │    │

Fa0/1 Fa0/2 Fa0/3 Fa0/4

│    │    │    │

▼    ▼    ▼    ▼

Laptop TV   Phone Friend's

V10   V10   V20    Phone V20

Device and VLAN Table

| Device | Switch Port | VLAN | Purpose |
| --- | --- | --- | --- |
| Laptop | Fa0/1 | VLAN 10 | Streaming |
| Smart TV | Fa0/2 | VLAN 10 | Streaming |
| Phone | Fa0/3 | VLAN 20 | Personal |
| Friend's Phone | Fa0/4 | VLAN 20 | Personal |

Important point

Devices in different VLANs are separated at Layer 2.

For example:

Laptop VLAN 10  ──X──  Phone VLAN 20

They cannot communicate directly through the switch.

To allow communication between VLANs, you need a router or Layer 3 switch with appropriate routing configuration.

2. Configure Two VLANs in Cisco Packet Tracer

Objective

Create:

- VLAN 10 – STREAMING

- VLAN 20 – PERSONAL

Connect:

- 3 PCs to VLAN 10

- 2 PCs to VLAN 20

Then verify that devices in different VLANs cannot ping each other.

A. Create the topology

In Cisco Packet Tracer, add:

- 1 × Cisco 2960 switch

- 5 × PCs

Connect them using Copper Straight-Through cables.

Example connections

PC1 ───── Fa0/1

PC2 ───── Fa0/2

PC3 ───── Fa0/3

PC4 ───── Fa0/4

PC5 ───── Fa0/5

│

▼

SWITCH

VLAN assignment

VLAN 10 – STREAMING

PC1 → Fa0/1

PC2 → Fa0/2

PC3 → Fa0/3

VLAN 20 – PERSONAL

PC4 → Fa0/4

PC5 → Fa0/5

B. Configure VLAN 10 and VLAN 20

Click the switch → CLI.

Enter:

enable

configure terminal

Create VLAN 10

vlan 10

name STREAMING

exit

Create VLAN 20

vlan 20

name PERSONAL

exit

Now the switch has two VLANs.

C. Assign Three PCs to VLAN 10

We need ports:

Fa0/1

Fa0/2

Fa0/3

to belong to VLAN 10.

Enter:

interface range fastEthernet 0/1-3

switchport mode access

switchport access vlan 10

exit

Explanation

interface range fastEthernet 0/1-3

Selects ports Fa0/1 through Fa0/3.

switchport mode access

Makes the ports access ports.

switchport access vlan 10

Assigns those ports to VLAN 10.

D. Assign Two PCs to VLAN 20

Ports:

Fa0/4

Fa0/5

Enter:

interface range fastEthernet 0/4-5

switchport mode access

switchport access vlan 20

exit

Now:

PC1 → VLAN 10

PC2 → VLAN 10

PC3 → VLAN 10

PC4 → VLAN 20

PC5 → VLAN 20

E. Configure IP Addresses

For a simple Layer-2 VLAN isolation test, give devices in each VLAN addresses from different IP subnets.

VLAN 10

| PC | IP Address | Subnet Mask |
| --- | --- | --- |
| PC1 | 192.168.10.10 | 255.255.255.0 |
| PC2 | 192.168.10.11 | 255.255.255.0 |
| PC3 | 192.168.10.12 | 255.255.255.0 |

VLAN 20

| PC | IP Address | Subnet Mask |
| --- | --- | --- |
| PC4 | 192.168.20.10 | 255.255.255.0 |
| PC5 | 192.168.20.11 | 255.255.255.0 |

For this isolated test, you do not need to configure a default gateway.

F. Test PCs Within the Same VLAN

From PC1, open:

Desktop → Command Prompt

Run:

ping 192.168.10.11

PC1 should be able to communicate with PC2 because both are in VLAN 10.

Also test:

ping 192.168.10.12

This should also succeed.

Similarly, from PC4:

ping 192.168.20.11

should succeed because PC4 and PC5 are both in VLAN 20.

G. Test Communication Between Different VLANs

From PC1:

ping 192.168.20.10

PC1 is in:

VLAN 10

192.168.10.0/24

PC4 is in:

VLAN 20

192.168.20.0/24

Without a router or Layer-3 switch providing inter-VLAN routing, the ping should fail.

Expected result

PC1 VLAN 10

│

│ Ping

X

│

PC4 VLAN 20

You may see:

Request timed out.

or:

Destination host unreachable.

The exact message depends on the Packet Tracer setup.

H. Verify VLAN Configuration

On the switch, run:

show vlan brief

You should see something similar to:

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------

1    default                          active    Fa0/6, Fa0/7, ...

10   STREAMING                        active    Fa0/1, Fa0/2, Fa0/3

20   PERSONAL                         active    Fa0/4, Fa0/5

This is important evidence for your assignment.

3. Move a PC from VLAN 10 to VLAN 20

Objective

Initially, PC3 belongs to VLAN 10.

We will move it to VLAN 20 and test the change.

Before

PC3

│

Fa0/3

│

VLAN 10 – STREAMING

After

PC3

│

Fa0/3

│

VLAN 20 – PERSONAL

A. Before Configuration

Suppose PC3 is connected to:

Fa0/3

Check the configuration:

show vlan brief

You should see:

10   STREAMING    active    Fa0/1, Fa0/2, Fa0/3

20   PERSONAL     active    Fa0/4, Fa0/5

Therefore:

PC3 → Fa0/3 → VLAN 10

B. Test Before Moving

Suppose:

PC3 = 192.168.10.12

PC1 = 192.168.10.10

PC4 = 192.168.20.10

From PC3:

ping 192.168.10.10

This should work because both are in VLAN 10.

Now test PC4:

ping 192.168.20.10

This should fail because PC3 is in VLAN 10 and PC4 is in VLAN 20, with no inter-VLAN router configured.

C. Move PC3 to VLAN 20

Go to the switch CLI.

enable

configure terminal

Select Fa0/3:

interface fastEthernet 0/3

Assign it to VLAN 20:

switchport mode access

switchport access vlan 20

exit

Now Fa0/3 belongs to VLAN 20.

D. Verify the Change

Run:

show vlan brief

You should now see something like:

10   STREAMING    active    Fa0/1, Fa0/2

20   PERSONAL     active    Fa0/3, Fa0/4, Fa0/5

The important change is:

Before:

VLAN 10 → Fa0/3

After:

VLAN 20 → Fa0/3

E. Change PC3's IP Address

Because PC3 has moved from VLAN 10 to VLAN 20, it should use the VLAN 20 IP subnet.

Change PC3 from:

192.168.10.12

to:

192.168.20.12

Subnet mask:

255.255.255.0

The final arrangement becomes:

VLAN 20

│

├── PC3 → 192.168.20.12

├── PC4 → 192.168.20.10

└── PC5 → 192.168.20.11

F. Test After the Change

From PC3:

ping 192.168.20.10

This should now work because:

PC3 → VLAN 20

PC4 → VLAN 20

Also test:

ping 192.168.20.11

This should work as well.

Before and After Table

| Test | Before | After |
| --- | --- | --- |
| PC3 VLAN | VLAN 10 | VLAN 20 |
| PC3 IP | 192.168.10.12 | 192.168.20.12 |
| Ping PC1 | Works | Normally fails without inter-VLAN routing |
| Ping PC4 | Fails | Works |
| Switch port | Fa0/3 → VLAN 10 | Fa0/3 → VLAN 20 |

4. Assign 10 Switch Ports to VLAN 30 Using a Range Command

Objective

Assign 10 ports to VLAN 30 without writing the configuration ten times.

We will use Cisco IOS / Packet Tracer syntax.

Step 1 – Create VLAN 30

enable

configure terminal

vlan 30

name STAFF

exit

Step 2 – Select 10 ports using interface range

For ports Fa0/1 through Fa0/10:

interface range fastEthernet 0/1-10

This selects all ten ports at once.

Step 3 – Configure them as access ports

switchport mode access

Step 4 – Assign VLAN 30

switchport access vlan 30

Complete Configuration

enable

configure terminal

vlan 30

name STAFF

exit

interface range fastEthernet 0/1-10

switchport mode access

switchport access vlan 30

exit

end

What happens?

Instead of writing:

Fa0/1 → VLAN 30

Fa0/2 → VLAN 30

Fa0/3 → VLAN 30

...

Fa0/10 → VLAN 30

we use:

interface range fastEthernet 0/1-10

and configure all ten ports together.

Verify the Configuration

Run:

show vlan brief

Expected result:

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------

1    default                          active    Fa0/11, Fa0/12, ...

30   STAFF                            active    Fa0/1, Fa0/2, Fa0/3,

Fa0/4, Fa0/5, Fa0/6,

Fa0/7, Fa0/8, Fa0/9,

Fa0/10

You can also check individual interfaces with:

show interfaces status

Complete Practical Summary

SWITCH

│

┌──────────────────┼──────────────────┐

│                  │                  │

VLAN 10             VLAN 20           VLAN 30

STREAMING            PERSONAL             STAFF

│                  │                  │

┌────┼────┐         ┌───┴───┐       Fa0/1-Fa0/10

│    │    │         │       │

PC1  PC2  PC3       PC4     PC5

Important Commands

| Purpose | Command |
| --- | --- |
| Enter privileged mode | enable |
| Enter configuration mode | configure terminal |
| Create VLAN | vlan 10 |
| Name VLAN | name STREAMING |
| Select one port | interface fa0/1 |
| Select multiple ports | interface range fa0/1-10 |
| Make access port | switchport mode access |
| Assign VLAN | switchport access vlan 10 |
| View VLANs | show vlan brief |
| View interfaces | show interfaces status |
| Test connectivity | ping <IP-address> |

Conclusion

VLANs allow a physical switch to be divided into separate logical networks. In this practical, VLAN 10 was used for streaming devices and VLAN 20 for personal devices. PCs within the same VLAN can communicate at Layer 2, while communication between different VLANs requires inter-VLAN routing through a router or Layer 3 switch. A PC can be moved between VLANs simply by changing its switch-port VLAN assignment. Finally, the interface range command makes it possible to configure multiple switch ports efficiently, such as assigning ten ports to VLAN 30 at once.