TCP/IP SuiteTheory

TCP/IP Model and Wireshark Practical Assignment — Detailed Answers

1. Capture Flipkart traffic in Wireshark and identify four layers

Objective: To capture network packets while refreshing the Flipkart homepage and identify one packet or protocol example for each TCP/IP layer by examining its headers.

Practical procedure

- Open Wireshark on your laptop.

- Select your active Wi-Fi or Ethernet network interface.

- Click Start Capturing Packets.

- Open your browser and visit https://www.flipkart.com.

- Refresh the homepage using Ctrl + R and wait a few seconds.

- Return to Wireshark and click the red Stop button.

- Select any packet and expand the packet details in the middle pane to inspect its headers.

You can use these display filters to find relevant packets:

| TCP/IP layer | Wireshark display filter | Header or protocol to inspect |
| --- | --- | --- |
| Application | dns or http | DNS query or HTTP request, if present |
| Transport | tcp or udp | TCP or UDP header |
| Internet | ip or ipv6 | Source and destination IP addresses |
| Network Access | eth or wlan | Ethernet or Wi-Fi frame header |

Note: Flipkart uses HTTPS, so your capture may show TLS, TCP, QUIC, DNS, IPv4/IPv6 and Ethernet/Wi-Fi rather than readable HTTP. If QUIC is used, the transport protocol is UDP. DNS might not appear if the result is cached.

Example observations for your practical file

These are examples of what you might record. Replace them with the actual values from your capture.

| Layer | Example protocol | What to look for | Function |
| --- | --- | --- | --- |
| Application | DNS | Domain name query for Flipkart | Resolves a domain name to an IP address |
| Transport | TCP | Source port, destination port, flags | Manages connection and data delivery |
| Internet | IPv4 | Source IP, destination IP, TTL | Addresses and routes packets between networks |
| Network Access | Ethernet | Source MAC, destination MAC, EtherType | Transfers frames over a local network |

Result: Wireshark allows us to examine different protocol headers involved in loading the Flipkart homepage.

2. Compare the OSI Model and TCP/IP Model

The OSI Model has seven layers, while the TCP/IP Model commonly has four layers. Both models explain how data travels from one device to another over a network.

| OSI Model (7 layers) | TCP/IP Model (4 layers) | Real-world example |
| --- | --- | --- |
| 7. Application | Application | WhatsApp messages, Flipkart browsing |
| 6. Presentation | Application | Instagram image formats, encryption |
| 5. Session | Application | Maintaining an application login session |
| 4. Transport | Transport | TCP reliable delivery, UDP streaming |
| 3. Network | Internet | IP addressing and routing |
| 2. Data Link | Network Access | Wi-Fi frames and Ethernet MAC addresses |
| 1. Physical | Network Access | Wi-Fi radio signals and Ethernet cables |

Main differences

| Feature | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Number of layers | 7 | 4 |
| Developed by | ISO | DARPA research community; developed through the DoD networking program |
| Main purpose | Reference model for understanding network communication | Practical model for Internet communication |
| Session and Presentation | Separate layers | Included in the Application layer |
| Data Link and Physical | Separate layers | Combined into the Network Access layer |
| Common use | Learning, network design and troubleshooting | Real-world Internet networking |

Conclusion: The OSI Model provides a detailed seven-layer framework, whereas the TCP/IP Model groups related functions into four layers and describes the architecture used by the Internet.

3. TCP segment header: fields and their functions

A TCP segment is a unit of data containing a TCP header and, optionally, application data. The TCP header belongs to the Transport layer (Layer 4) of the OSI model and the Transport layer of TCP/IP.

Use this labeled example for your assignment. The values are illustrative; your trainer's screenshot may contain different values.

TCP Header Structure

Source Port

Identifies the sending application's port.

Destination Port

Identifies the receiving application's port.

Sequence Number

Tracks the position of data bytes.

Acknowledgment Number

Indicates the next byte expected from the other side.

Data Offset

Specifies the TCP header length.

Reserved

Reserved bits for protocol use.

Flags

Control connection setup, acknowledgment and closure.

Window Size

Advertises how much data the receiver can accept.

Checksum

Detects corruption in the TCP header and data.

Urgent Pointer

Marks urgent data when the URG flag is set.

Options + Padding

Carries optional settings and aligns the header.

Explanation of important TCP fields

| Field | Function |
| --- | --- |
| Source Port | Identifies the port used by the sending application. |
| Destination Port | Identifies the port used by the receiving application or service. |
| Sequence Number | Helps arrange data bytes in the correct order and supports retransmission. |
| Acknowledgment Number | Indicates the next byte expected when acknowledgment is being used. |
| Flags | SYN starts a connection, ACK acknowledges data, FIN closes a connection gracefully, and RST resets a connection. |
| Window Size | Supports flow control by advertising the receiver's available capacity. |
| Checksum | Helps detect corruption in the TCP header and payload. |
| Urgent Pointer | Indicates the end of urgent data when the URG flag is set. |
| Data Offset | Tells the receiver where the TCP payload begins. |
| Options | Can carry settings such as Maximum Segment Size (MSS) and window scaling. |

Note: A standard TCP header is at least 20 bytes long. With options, it can be up to 60 bytes long.

4. Diagram: message travelling from your device to Instagram's server

The diagram below shows how a message moves through the TCP/IP networking process. The OSI layers are included in brackets to connect the two models.

Your device — Instagram app

You type a message or upload a photo.

1. Application Layer (OSI 7–5)

Instagram prepares the request, including the content and application information.

2. Transport Layer (OSI 4)

TCP divides the data into segments when TCP is used. UDP carries datagrams when a UDP-based transport is used.

3. Internet Layer (OSI 3)

IP addressing and routing carry packets toward the destination server.

4. Network Access Layer (OSI 2–1)

The data is framed for the local link and transmitted as Wi-Fi radio signals, mobile signals or wired signals.

Internet → Instagram server

Network equipment forwards the packets. The server processes the request and sends a response.

Step-by-step explanation

- Application: You type a message, comment or upload a photo using Instagram.

- Transport: The application communicates using the transport protocol selected by its networking stack. TCP adds fields such as source port, destination port and sequence number when TCP is used.

- Internet: IP addresses identify the packet's source and destination. Routers forward packets toward the destination.

- Network Access: Your device puts the packet into a local network frame and transmits it over Wi-Fi or a mobile network.

- Server: Instagram's server receives and processes the request. Response data returns through the network, and your device passes it to the app.

Important: Instagram's exact protocols depend on the feature and connection. This is a conceptual diagram; not every Instagram operation necessarily uses TCP.

5. Analyze one packet captured in Wireshark

Objective: Select one packet from your Flipkart capture, identify its TCP/IP layer, explain how you recognized it, and connect it to a real-world application action.

Example packet analysis

Suppose you captured a packet with these details:

| Field | Example observation |
| --- | --- |
| Protocol | HTTP |
| Source | Your laptop's IP address |
| Destination | Flipkart server's IP address |
| Info | GET / HTTP/2 or a similar web request |
| TCP destination port | Commonly 443 for HTTPS traffic |

These are illustrative observations. Your actual capture might show TLS, TCP, QUIC or another protocol.

Written analysis

Packet selected: An HTTP request packet from a browser accessing Flipkart.

TCP/IP layer: Application layer.

How I identified it: I selected the packet in Wireshark and checked the Protocol and Info columns. The HTTP protocol and request information indicated that the browser was requesting web content. I expanded the packet details to inspect the HTTP fields.

Real-world action: This packet represents my browser requesting the Flipkart homepage after I entered the website address or refreshed the page. The server processes the request and returns content that the browser uses to display the homepage.

Conclusion: Wireshark helps identify network protocols and examine packet headers. By inspecting application, transport, Internet and network-access information, we can understand how web requests travel across a network.

Alternative if your capture does not show HTTP

If Wireshark shows encrypted HTTPS traffic, choose a packet you can actually identify:

- TLS: Application data protection and secure communication setup.

- TCP: Connection establishment or data delivery.

- IPv4/IPv6: Packet addressing and routing.

- Ethernet/Wi-Fi: Local frame delivery.

For your final submission, use the real packet's protocol and Info details rather than copying the illustrative HTTP example unchanged.