Wireless Networking Pending

Wi-Fi Standards, Home Wi-Fi Connection & SSID Hiding

1. Three Popular Wi-Fi Standards

Wi-Fi standards are developed by the IEEE 802.11 family. Different standards provide different speeds, frequency bands, and features.

| Wi-Fi Standard | Common Name | Frequency Band | Maximum Theoretical Speed* | Key Difference |
| --- | --- | --- | --- | --- |
| 802.11n | Wi-Fi 4 | 2.4 GHz / 5 GHz | Up to 600 Mbps | Works on both 2.4 GHz and 5 GHz |
| 802.11ac | Wi-Fi 5 | Mainly 5 GHz | Up to ~6.9 Gbps | Faster 5 GHz Wi-Fi with wider channels and better MIMO |
| 802.11ax | Wi-Fi 6 | 2.4 GHz / 5 GHz | Up to ~9.6 Gbps | Better efficiency and performance with many connected devices |

*Theoretical maximums depend on channel width, number of antennas/spatial streams, modulation, and device capability. Real-world speeds are normally lower.

802.11n – Wi-Fi 4

- Introduced MIMO technology to improve wireless performance.

- Can operate on 2.4 GHz and 5 GHz.

- Has better range than many 5 GHz-only configurations.

- Maximum theoretical speed can reach approximately 600 Mbps under ideal conditions.

Example:
A laptop connected using 802.11n may get good coverage in a home, but its actual Internet speed depends on the router, ISP, interference, and device.

802.11ac – Wi-Fi 5

- Primarily operates on the 5 GHz band.

- Provides significantly higher speeds than 802.11n.

- Supports wider channels such as 80 MHz and, in some implementations, 160 MHz.

- Uses improved MIMO technologies.

Main difference:
802.11ac generally provides higher throughput than 802.11n, especially on 5 GHz.

802.11ax – Wi-Fi 6

- Works on 2.4 GHz and 5 GHz.

- Designed not only for high speed but also for efficient communication when many devices are connected.

- Uses technologies such as OFDMA and improved MU-MIMO.

- Can provide up to approximately 9.6 Gbps theoretical aggregate PHY rate.

Main difference:
Wi-Fi 6 is particularly useful in environments with many devices, such as homes, colleges, offices, and public places.

Simple comparison

802.11n  → Wi-Fi 4 → 2.4/5 GHz → Good coverage

802.11ac → Wi-Fi 5 → 5 GHz     → Higher speed

802.11ax → Wi-Fi 6 → 2.4/5 GHz → High efficiency + many devices

2. Smartphone Connecting to Internet Through Home Wi-Fi

Simple Network Diagram

You can draw this diagram on paper for your assignment:

INTERNET

│

│ Wired

│

┌──────▼──────┐

│    MODEM    │

│ /ONT        │

└──────┬──────┘

│

│ Ethernet Cable

│ (Wired)

┌──────▼──────┐

│ WI-FI ROUTER │

│              │

│  LAN/WAN     │

│  Wi-Fi AP    │

└──────┬──────┘

))))

Wi-Fi

(Wireless)

))))

┌──────▼──────┐

│  Smartphone  │

│              │

│ Wi-Fi Client │

└──────────────┘

Wireless and Wired Parts

Wired part:

Internet/ISP

│

│ Ethernet/Fiber

▼

Modem/ONT

│

│ Ethernet cable

▼

Wi-Fi Router

Wireless part:

Wi-Fi Router

)))))

)))))  Wi-Fi radio signal

)))))

▼

Smartphone

How the Connection Works

Step 1 – ISP provides Internet

Your Internet Service Provider (ISP) provides an Internet connection to your home.

For example:

ISP → Fiber/Internet connection → Home

Step 2 – Modem/ONT receives the connection

Depending on the ISP technology, a modem or optical network terminal (ONT) connects the home network to the ISP.

Step 3 – Router creates the home network

The Wi-Fi router:

- Connects the home network to the Internet.

- Assigns local IP addresses using DHCP.

- Provides Wi-Fi.

- Provides Ethernet LAN ports.

- Performs routing and usually NAT/firewall functions.

Step 4 – Smartphone connects wirelessly

The smartphone searches for available Wi-Fi networks and connects to the selected SSID.

For example:

SSID: Home_WiFi

Security: WPA2/WPA3

Password: ********

The smartphone then receives an IP address, such as:

192.168.1.25

Step 5 – Smartphone accesses the Internet

When you open YouTube, Instagram, Google, etc.:

Smartphone

↓

Wi-Fi

↓

Home Router

↓

ISP

↓

Internet

↓

Website/App Server

The response travels back through the network to your smartphone.

3. Scan Available Wi-Fi Networks

Important Practical Note

This part requires an actual scan on your laptop or phone. I cannot see the Wi-Fi networks around you, so you should record the real SSIDs and signal strengths from your device rather than using made-up results.

Windows 11

Click:

Wi-Fi icon → Wi-Fi network list

You can record:

- SSID

- Signal strength

- Security type, if Windows displays it

You can also use Command Prompt:

netsh wlan show networks mode=bssid

This can show nearby wireless networks and information such as:

SSID

Signal

Radio type

Channel

Authentication

Encryption

Example Observation Table

Use your actual results:

| No. | SSID | Signal Strength | Security | My Choice |
| --- | --- | --- | --- | --- |
| 1 | Home_WiFi | 90% | WPA3-Personal | ✅ Best |
| 2 | Neighbor_WiFi | 65% | WPA2-Personal | Good |
| 3 | Public_WiFi | 40% | Open | ❌ Avoid |

These are example values only. Replace them with your actual scan results.

How to Choose the Best Network

Two important factors are:

1. Signal strength

A stronger signal generally gives a more stable connection.

For example:

90% → Excellent

70% → Good

50% → Fair

30% → Weak

These are rough practical categories, not universal thresholds.

2. Security

Prefer:

WPA3 > WPA2 > Open network

WPA3 provides newer security protections than WPA2 when supported by both the router and device.

Avoid connecting to an unknown open Wi-Fi network for sensitive activities such as:

- Online banking

- Entering passwords

- Shopping

- Accessing important accounts

Example Selection Explanation

You can write:

I would choose Home_WiFi because it has a strong signal and uses WPA3 security. A strong signal provides a more stable connection, while WPA3 provides better modern wireless security. I would avoid an open Wi-Fi network because it does not require a Wi-Fi password and provides less protection at the wireless-access level.

If your actual scan shows WPA2 instead of WPA3, you can write:

I would choose the network with the strongest reliable signal among the networks using WPA2 or WPA3 security.

4. SSID Hiding

What is an SSID?

SSID stands for:

Service Set Identifier

In simple words, the SSID is the name of a Wi-Fi network that appears when you search for Wi-Fi.

Example:

Available Wi-Fi Networks

📶 Home_WiFi

📶 College_WiFi

📶 JioFiber_1234

📶 TP-Link_5678

Here, Home_WiFi is the SSID.

What is SSID Hiding?

SSID hiding is a router setting that prevents the Wi-Fi network name from being included in normal Wi-Fi beacon advertisements.

When SSID broadcast is enabled:

Phone

↓

Scan Wi-Fi

↓

Home_WiFi appears

When SSID broadcast is disabled:

Phone

↓

Scan Wi-Fi

↓

Home_WiFi may not appear normally

The user can usually connect by manually entering the network name and password.

Simple diagram

SSID Broadcast ON

Router

│

├── SSID: Home_WiFi

│

▼

Phone sees: Home_WiFi

With SSID broadcast disabled:

SSID Broadcast OFF

Router

│

└── Network name not advertised normally

│

▼

User manually

enters SSID

Advantage of Hiding SSID

Advantage: Less visible to casual users

The network name does not normally appear in the standard Wi-Fi network list.

For example:

Normal:

Home_WiFi

Office_WiFi

Guest_WiFi

Hidden:

Home_WiFi is not normally shown

This can reduce accidental connection attempts or make the network less obvious to casual users.

However, hiding the SSID is not strong security.

Disadvantage of Hiding SSID

Disadvantage: It does not make the network truly secure

A hidden SSID can still be discovered by someone using wireless-analysis tools because Wi-Fi devices exchange management traffic.

Therefore:

SSID Hidden ≠ Secure Wi-Fi

The real protection should come from:

- WPA2/WPA3 encryption

- Strong Wi-Fi password

- Updated router firmware

- Disabled WPS when appropriate

- Separate guest network for visitors

- Regularly changing compromised passwords

Another disadvantage

Connecting devices can become less convenient because the user may need to manually enter the SSID.

SSID Hiding – Short Comparison

| Feature | SSID Visible | SSID Hidden |
| --- | --- | --- |
| Network name appears normally | Yes | No |
| Easy for users to find | Yes | No |
| Prevents casual users from seeing name | No | Yes |
| Strong security by itself | No | No |
| Recommended as main security method | No | No |
| WPA2/WPA3 still required | Yes | Yes |

Final Assignment Conclusion

Wi-Fi standards such as 802.11n, 802.11ac, and 802.11ax provide different levels of speed, frequency support, and efficiency. A home Wi-Fi network normally connects the smartphone wirelessly to the router, while the router uses a wired connection to the modem/ONT and ISP. When selecting a Wi-Fi network, both signal strength and security should be considered, with WPA2/WPA3 networks generally preferred over open networks. SSID hiding can make a Wi-Fi network less visible to casual users, but it should not be considered a security mechanism; strong WPA2/WPA3 authentication and a strong password are much more important.

Top of Form

Bottom of Form